Appendix No. 1 to the STC

Ref: 26ZAK002

DETAILED DESCRIPTION OF THE SUBJECT OF THE CONTRACT (“DDoSoC”)

THE SUBJECT OF THE CONTRACT TITLED:

THE INSPECTIONS OF DOWNHOLE EQUIPMENT (DHSV) OF THE PRODUCTION CAVERN WELLS IN THE MOGILNO CUGS AND KOSAKOWO CUGS

PROCEDURE NO.: 26ZAK002

- CONTENTS

1	GENERAL DESCRIPTION OF THE SUBJECT OF THE CONTRACT	3

2	UNITS COVERED BY THE SUBJECT OF THE CONTRACT	3

2.1	SPECIFICATION OF WELLS IN STORAGE CAVERNS	4

2.2	SPECIFICATION OF DOWNHOLE SAFETY VALVES	4

3	DETAILED SCOPE OF THE SUBJECT OF THE CONTRACT	5

3.1	SERVICE ACTIVITIES – BASIC WORKS	6

3.1.1	Periodic inspections	7

3.1.2	Inspection and assessment of technical condition	10

3.1.3	Post-completion documentation (acceptance documentation)	10

3.2	SERVICE ACTIVITIES – ADDITIONAL WORKS	11

3.2.1	Additional works – Non-standard services	12

3.2.2	Additional works – Repairs of the downhole safety valve	14

3.2.3	Additional works – Assembly and disassembly of mechanical plugs in the well	15

4	METHOD AND TIME OF ORDER DELIVERY	19

5	OTHER MATERIAL CONDITIONS OF THE CONTRACT	20

5.1	OBLIGATIONS OF THE CONTRACTING ENTITY	20

5.2	SPECIFIC REQUIREMENTS FOR THE CONTRACTOR	21

5.2.1	REQUIREMENTS CONCERNING SPECIALIST EQUIPMENT	22

6	APPENDIX	24

# GENERAL DESCRIPTION OF THE SUBJECT OF THE CONTRACT

The Subject of the Contract consists in the provision of inspection services for downhole equipment of production caverns for the process infrastructure of:

- The Mogilno Cavern Underground Gas Storage Facility (Mogilno CUGS)

KPMG Mogilno w Palędziu Dolnym

Palędzie Dolne, 88-300 Mogilno, woj. kujawsko-pomorskie

- The Kosakowo Cavern Underground Gas Storage (Kosakowo CUGS)

KPMG Kosakowo w Dębogórzu

ul. Rumska 28; 81-198 Dębogórze; woj. pomorskie

The subject matter of the contract comprises successive orders for SERVICE OPERATIONS (the “Services”).

  - The SERVICES concerning the units / installations covered by Subject of theContract include:

    - BASIC WORKS in respect of the scheduled and preventive maintenance activities planned with respect to the following tasks:

  - Periodic inspections (Preventive maintenance),

  - Checking and assessment of technical condition,

  - Post-completion documentation (acceptance documentation).

    - ADDITIONAL WORKS – activities covered by the right of option referred to in the STC, concerning unplanned (non-standard) interventions with respect to the following tasks:

  - Non-standard services, in such case, the Contracting Entity shall exercise the right of option in the event of needs arising from post-inspection recommendations and identified failures or damage;

  - Repair of the downhole safety valve, in this case the Contracting Entity will exercise the right of option in the event of equipment failure;

  - Installation and removal of mechanical plugs in the borehole – in this case the Contracting Entity will exercise its right of option in the event of intervention works related to the replacement of surface fittings or the performance of leakage tests on the production casing.

# UNITS COVERED BY THE SUBJECT OF THE CONTRACT

The units covered by the Subject of the Contract are installed in the subsurface part of the process facilities of the Contracting Entity at the following locations:

- KPMG Mogilno w Palędziu Dolnym

Palędzie Dolne, 88-300 Mogilno, woj. kujawsko-pomorskie

- KPMG Kosakowo w Dębogórzu

ul. Rumska 28; 81-198 Dębogórze; woj. Pomorskie

and are necessary for the purposes of underground storage of gaseous fuel within the process infrastructure of the CUGS Mining Plant. The gaseous fuel is high-methane natural gas.

The installation concerned by the Subject of the Contract consists of the downhole completion components of storage caverns, which include in particular:

- production casing and tubing assembly together with technical components,

- a packer sealing the annulus,

- downhole safety valve,

which are supported by surface infrastructure, i.e. the wellhead (see the diagram in Annex 1.2).

## SPECIFICATION OF WELLS IN STORAGE CAVERNS

The storage cavern wells covered by the Subject of the contract include:

- For the location of Mogilno CUGS – 14 storage cavern wells identified as:

Z-1, Z-2, Z-3, Z-4, Z-5, Z-6, Z-7, Z-8, Z-9, Z-11, Z-12, Z-15, Z-16, Z-17,

as specified in Annex 1.1.A.

- For the location of Kosakowo CUGS – 10 storage cavern wells identified as:

K-1, K-2, K-3, K-4, K-5, K-6, K-7, K-8, K-9, K-10,

as specified in Annex 1.1.B.

Annexes 1.1.A and 1.1.B to the DDoSoC include tables  presenting the technical data as at the date of preparing the specifications of individual wells of natural gas storage caverns for individual locations, taking into account the casing, downhole completion, cavern geometry, maximum wellhead gas pressure, type of wellhead.

As part of the order to be placed under the agreement, the Contracting Entity shall define the unit / facility using the above mentioned identification details for the respective storage caverns.

## SPECIFICATION OF DOWNHOLE SAFETY VALVES

The specialist equipment being subject to the specifications and used in the storage cavern wells which is covered by the Subject of the Contract includes Halliburton downhole safety valves (Storm Choke):

- WLRSV.OO. 5.813/2.813/H2S/5K/CAC - 11 pieces. (plus back up –2 pieces)

- WLRSV.OO. 7.05/3.813/STD/5K/CAC - 13 pieces (plus back up – 1 piece).

The specification of the downhole safety valves installed at CUGS locations is presented the following table:

Table – Specification of downhole safety valves

| Parameter | WLRSV.OO 5.813, 2.813 | WLRSV OO 7.05, 3.813 |
| --- | --- | --- |
| Valve model | OO | OO |
| closure type | POPPET | POPPET |
| Size | 7” | 7.05” |
| Minimum diameter ID | 2.813” | 3.813” |
| Maximum diameter OD | 5.803” | 7.040” |
| Length | 47.94” | 49.20” |
| Upper sub ID | 3.51” | 5.34” |
| Top thread | 4 7/8 OTIS SLB | 6-8 SLB |
| Type of connection | BOX | BOX |
| Pressure range | 5000 lbs/sq in | 5000 lbs/sq in |
| Temperature range | 20-300 F | 20-300 F |
| Dedicated for landing nipple | RNT 5.813 | 7.05 RN LN |
| Landing nipple profile | RP 2.813 | RP 3.813 |
| Size | 5.25 | 6 |
| Lock (Landing nipple) | 5.813 | 7.05 |
| Landing nipple profile | RNT | RN |
| Minimum diameter ID | 3.5” | 5.25” |
| Length | 17.17” | 17.17” |
| Fishing neck ID | 4” | 5.880” |
| Bottom thread | 4 7/8”-8 OTIS SLB PIN | 6-8 OTIS SLB |
| Pulling tool | 40GR45600 | 40GR70500 |
| Running tool | 41RXN52550 | 41RNT70550 |

Detailed documentation of the downhole safety valve equipment for individual storage caverns can be obtained from the Contracting Entity.

The Contracting Entity shall make the downhole safety valve documentation available within 20 days of the execution of the agreement.

The Contracting Entity provides for the possibility of rotating the downhole safety valves, provided that the selection and specification requirements for a given storage cavern are maintained.

The downhole safety valve was selected at the engineering design stage and makes part of completed deliveries, based on a customised approach to the assumed technical and quality parameters (working pressure, working temperature, maximum gas flow, presence of H2S).

# DETAILED SCOPE OF THE SUBJECT OF THE CONTRACT

The Subject of the Contract includes specifically:

- The inspection (maintenance schedule) of the downhole completion components of production caverns for the purposes of the delivery of the Subject of the Contract.

- The presentation, for the Contracting Entity’s approval, of a detailed schedule of works in the scope of the Services required for the execution of the task – within 2 weeks of order placement.

- The use of specialized tools and equipment that fully guarantee compatibility with surface (connection) and subsurface infrastructure while ensuring the performance of the Subject of the Contract in accordance with the requirements of the Contracting Entity (in particular the acceptable technical and quality parameters) and the applicable standards of health and safety and environmental protection.

- The definition and consultation of the inspection methodology for the purposes of equipment selection and review at the tender submission stage.

- The Contractor shall be required to select all sub-assemblies and components (parts of elements) so as to ensure their full compatibility and the assumed original technical and quality parameters of the unit / facility within the scope of the Services (dedicated for a given storage cavern).

- The performance of the scope of work for the identified inspection activities within the period agreed with the Contracting Entity. The completion time of a planned task concerning field work activities for one storage cavern cannot exceed 5 business days for operations involving the disassembly of a downhole safety valve, inspection of the downhole completion components and the cavern neck with a downhole video camera, measurement of the gas storage cavern floor and the assembly of the downhole safety valve. Planned starting date and performance of the works to be agreed with the Contracting Entity.

- Installation of parts and sub-assemblies.

- Performance of a functional/pressure test and commissioning of the units.

- The preparation of post-completion documentation in the form of reports for a given Service.

During site works, the personnel of the Contractor (or its subcontractor) shall be required to comply with any restrictions resulting from the organization of works, which have been implemented and are applicable in the Contracting Entity’s organisation.

All site works shall be conditional on their performance exclusively with the use of the Contractor’s specialist equipment, tools and the equipment conforming to the requirements set out in in section 5.2 of the DDoSoC as well as all the requirements for obtaining a permit to operate and those from the law.

## SERVICE ACTIVITIES – BASIC WORKS

These works include the servicing of the downhole equipment components (safety valves) installed in the wells of the CUGS caverns at depths ranging from 560m to 1350 m.

As part of the Services, the Contractor shall ensure all the necessary materials, resources and personnel to carry out the Order. The works to be carried out in respect to the planned activities related to the inspection of the downhole equipment must guarantee full safety of their execution, in particular when performed in an active installation under the pressure of gas storing in storage.

The service works carried out in the field (on site) should be carried out in a way that minimizes downtime and complete unavailability of the storage cavern. In the event of circumstances preventing field works from being carried out within the agreed time, the Contractor shall be required to justify and indicate the reasons why the timely completion of the works is impossible.

The Services shall be provided by the Contractor on the basis of individual orders to be presented by the Contracting Entity to the Contractor according to the rules specified in the standard form of the agreement. Each time, the scope of Services shall be specified in the order.

### Periodic inspections

Periodic inspections shall be carried out on the basis of the requirements specified in the Operation and Maintenance Manuals (OMM) for the units / facilities covered by the Subject of the Contract and in accordance with the procedures of the equipment manufacturer for the installed downhole safety valves.

The materials and parts specified within the maintenance activities / required in the OMM shall be provided and installed by the Contractor as part of the inspection tasks and in accordance with the procedures of the equipment manufacturer for the installed safety valves.

The works shall be coordinated by persons assigned by the Contracting Entity to the supervision over service activities (facility works).

The field works (activities) related to:

      - disassembly of the downhole safety valve,

      - inspection of the downhole completion and cavern neck with a downhole video camera,

      - logging of the gas storage cavern total depth,

      - inspection and replacement of the downhole safety valve components,

      - assembly of the downhole safety valve,

shall be considered facility (site) works on a given CUGS location.

The duration of the above-mentioned site works for one storage cavern must not exceed 5 business days.

For the purposes of checking and inspection activities with respect to a downhole safety valve, the performance of the Services at the Contractor's location (in a specialist workshop indicated by the Contractor) shall be permitted.

#### Disassembly of the downhole safety valve

The Scope of Works for the downhole safety  valve disassembly service shall include in particular the following activities:

DETAILED DESCRIPTION

- mobilize the logging unit and a service crew to the location where the Service is to be carried out (including, in particular, the provision of the Slick Line unit ("S/L"), pressure control equipment, gauge, downhole safety valve pulling tools, sinker bars, spring jar, crew for supervision and execution of the works). - set up the slick line (S/L) unit with a spooled up steel wire, the pressure control equipment assembly top flange on wellhead. - place the assembly with a gauge in the pressure control equipment. - make up the pressure control equipment on the wellhead. - run in hole the gauge assembly and drift the casing to the landing depth of the downhole valve, pull out of the hole the gauge assembly. - replace with an assembly with a tool for pulling up the downhole safety valve. - running in hole and connecting the tool for pulling up the valve with a lock mandrel, - disassembly / pulling up the complete downhole valve. - disassembly the pressure control equipment and the S/L unit, disassembly top flange on wellhead. - demobilize the unit together with the crew and move to the location of the next task. - demobilize the unit together with the crew (return to base).

The works should be carried out when the downhole safety valve is being pulled up, with a single trip being allowed where the at pulling up rate must not exceed 30 meters per minute.

#### Inspection of the downhole completion components and the cavern neck using a downhole video camera

The Scope of Work for the inspection of the downhole completion components and the cavern neck using a downhole video camera shall include in particular the following activities:

DETAILED DESCRIPTION

- mobilize the logging unit and the crew to the location where the Service is to be carried out (including in particular the provision of the W/L unit, pressure control equipment, downhole video camera, sinker bars and crew for the supervision and execution of the works). - set up the logging unit with a cable spooled up on the winch and a downhole video camera, assembly top flange on wellhead. - place the downhole video camera in the pressure control equipment. - make up the pressure control equipment on the wellhead. - Run in hole the downhole video camera and carry out logging from the top to the bottom (video recording). - pull up the downhole video camera and carry out logging from the bottom to the top (video recording). - disassembly the pressure control equipment and the logging W/L and the downhole video camera assembly, disassembly top flange on wellhead. - demobilize the logging unit with the crew.

The works should be carried out when the assembly is being run into the hole, with a single trip being allowed. The running rate for the downhole video camera and video recording should be agreed with the Contracting Entity according to the following assumptions:

- to assess the technical condition of the casing the rate should not exceed 30 metres per minute, with the possibility of slowing down or stopping the process of running the camera into the hole for detailed assessment of technical condition.

- to assess the technical condition of the landing of the main downhole completion components (landing nipple, telescope sub, packer, etc.) – the rate should not exceed 2 metres per minute, with the possibility of slowing down or stopping the process of camera running into the hole for detailed assessment of technical condition.

- to assess the technical condition along the section down to the casing shoe – the rate should not exceed 3 metres per minute, with the possibility of slowing down or stopping the process of camera running into the hole for detailed assessment of technical condition.

- to assess the condition of the cavern neck – the rate should not exceed 3 metres per minute, with the possibility of slowing down or stopping the process of camera running into the hole for detailed assessment of the condition of the neck cavern.

As part of the inspection of the downhole equipment, video recording should be continued throughout the section being subject to the inspection. The video file shall be saved on digital media (flash drive) in two copies and handed over to the Contracting Entity.

#### Inspection of a downhole safety valve and replacement of its components

The Scope of Work for the inspection of a downhole safety valve and replacement of its components shall include in particular the following activities:

DETAILED DESCRIPTION

- clean the components of the downhole safety valve. - secure and transport the valve to the Contractor's workshop. - replace standard seals: - 7.05” valve (standard 7” Redress Kit per valve), - 5.813” valve (standard 5.813" Redress Kit per valve). - general inspection and test. - secure and transport the valve to the location where it is to be installed.

In case when it is necessary to install new components or parts, other than the ones specified above, such parts shall be replaced, subject to consultation with and approval by the Contracting Entity, within a separate scope of Service costs (in accordance with the price list set out in the Pricing Form or on the basis of a separate proposal in case when the part / service is not included in the specifications).

#### Logging of the total depth of cavern

The Scope of Work for logging of the gas storage cavern base shall include particular the following activities:

DETAILED DESCRIPTION

- mobilize the logging unit and the crew to the location where the service is to be carried out (including in particular the provision of the S/L unit, pressure control equipment, sinker bars, gauge/ sampler and crew for the supervision and execution of the works). - place the gauge / sampler in the pressure control equipment, assembly top flange on wellhead. - install the pressure control equipment on the x-tree. - run in hole gauge/ drift to the cavern base. - tag total depth / brine sampling, pulling up. - disassembly pressure control equipment, S/L unit and the gauge/ sampler assembly, disassembly top flange on wellhead. - demobilize the logging unit together with the crew and move to the location of the next task. - demobilize the logging unit together with the crew and return to base.

The works should be carried out when the logging tools are being run into the hole, with a single trip being allowed where the running rate must not exceed 50 meters per minute. In the final stage of running, the rate should be reduced to up to 3-4 metres per minute prior to reaching the expected total depth.

#### Installation of the downhole safety valve

The Scope of Works for the downhole safety valve installation service shall include in particular the following activities:

DETAILED DESCRIPTION

- mobilize the unit and service crew to the location where the Service is to be carried out. - set up the Slick Line with a spooled up steel wire, pressure control equipment, sinker bars, spring jar, tool for downhole valve running and setting, the downhole valve. - place the tool for running in and setting the downhole valve together with the downhole valve in the pressure control equipment, assembly top flange on wellhead. - install the pressure control equipment on the x-tree. - run the downhole valve and set the valve in the landing nipple, - pull out of the hole valve running and setting tool. - replace the tool for downhole running and setting with a tool with a valve shutting tool. - run in hole the valve shutting tool. - shut the downhole valve. - carry out valve pressure tests after reducing the wellhead pressure in accordance with the maintenance work programme. - pull up the valve shutting assembly. - disassembly the pressure control equipment and the wireline unit, disassembly top flange on wellhead. - demobilize the unit together with the crew and move to the location of the next task. - demobilize the unit together with the crew and return to base.

The works should be carried out when the valve assembly is being run into the hole, with a single trip being allowed where the at running rate must not exceed 30 meters per minute.

### Inspection and assessment of technical condition

The device will be handed over to the Contractor (handover document) to be taken to a specialist workshop for the inspection and assessment of technical condition assessment in respect of:

- checking the conformity of the fittings, their descriptions and markings, and any additions thereto,

- checking and specifying any damaged elements which need to be replaced,

- checking the condition of seals, non-destructive tests, etc.

- checking and tests to confirm the fitness for service and tightness of the valve,

- preparing the inspection results in the form of a service / inspection report.

In the event that any defects or damage are discovered during inspection activities, which could impair the re-installation of the unit (affect its proper functioning), the Con shall immediately inform the Contracting Entity about the need for additional Services.

The Contracting Entity shall be presented, in a digital form, with a detailed schedule of works in relation to the additional Services necessary to restore the full fitness of the equipment for service. The schedule should be sent to the following e-mail address: janusz.brozbar@gas-storage.pl, bogumil.augustyn@gas-storage.pl and to the indicated address of the contract coordinator. The deadline for providing the above information shall not exceed 7 days of date of acceptance of the valve to the workshop for inspection by the Contractor.

### Post-completion documentation (acceptance documentation)

For each completed scope of work, the Contractor shall prepare and present the Contracting Entity with documentation (in particular: official letter / report / statement / video recording) containing information on the scope of works carried out, materials and parts used, and shall attach reports from performed tests and other activities, together with recommendations, if applicable.

With respect to all the contracted work, the documentation to be compiled shall include:

- Report on the completed disassembly work on the safety valves,

- Report on the completed inspections of the casing downhole equipment (video),

- Report on the completed inspections of the safety valves (assessment and inspection),

- Report on the completed disassembly work on the downhole safety valves,

- Reports on works carried out for additional activities.

Reports on the field works shall be submitted to the Contracting Entity at the latest on the day following the completion of the works concerned.

The scope and form of the post-completion documentation must be agreed with and approved by the Contracting Entity.

The report on the completed Services should include:

  - Name of the Service provider and report number,

  - Name of the tested unit, its nameplate details and type,

  - Place of service of the tested unit,

  - Type and scope of measurements taken / works completed,

  - Date of their performance,

  - Name of the person who carried out the activities,

  - Details on the conditions under which the measurements / works were carried out,

  - Remarks, conclusions and recommendations resulting from visual inspections carried out in accordance with the requirements of the applicable standards, regulations and observations made during the installation and equipment checks.

The post-completion documentation for the completed Services must be handed over to the Contracting Entity not later than 30 days after completion of the Service.

For a video inspection, two copies should be provided to the Contracting Entity on a digital medium (flash disk) in the form of a video recording for the inspection of the downhole completion of the caver no later than on the day following the completion of the field work.

## SERVICE ACTIVITIES – ADDITIONAL WORKS

Additional activities – works and optional activities concerning unplanned intervention activities, the execution of which will be necessary to ensure the fitness for service and reliability of the operation of the downhole equipment components of the cavern. The works shall be carried out on the basis of an order in accordance with the agreement, according to the costs indicated in the pricing form.

As part of the additional activities, the Contractor shall ensure all the necessary tools, resources and personnel to carry out the task.

The works to be carried out with respect to additional activities must ensure the safety of their execution, in particular when performed in an active installation under the pressure of gas  storing in storage.

The service works carried out in the field (on site) should be carried out in a way that minimizes downtime and complete unavailability of the storage cavern. In the event of circumstances preventing field works from being carried out within the agreed time, the Contractor shall be required to justify and indicate the reasons why the timely completion of the works is impossible.

All site works must be carried out with the use of the Contractor 's specialist equipment conforming to the requirements specified in section 5.2 of the DDoSoC.

### Additional works – Non-standard services

NON-STANDARD SERVICES – additional (unscheduled) works which are listed in the table below shall be carried out on a contract basis, if needed, according to the post-inspection recommendations and the identified malfunctions/defects.

The deadline for the completion of activities related to the additional works (non-standard Services) shall be agreed with the Contracting Entity, but shall not exceed 30 days from the date of ordering the Service (contingent on the supply of parts and materials).

With respect to such non-standard activities, the following modus operandi shall apply:

a)	notification of repairs:

•	by email: CUGS Manager or Chief Operating Specialist, name.surname@gas-storage.pl

•	the Contractor shall be obligated to provide contact details (phone/email).

b)	time of acceptance of Service activities from the moment of notification – up to 5 business days of the notification by email,

c)	declared time of Service completion: to be agreed with the Contracting Entity – up to 30 days or other deadline agreed with the Contracting Entity, provided however that it does not exceed the guaranteed delivery time of spare parts – up to 20 weeks.

As part of non-standard Services, the Contractor shall provide all the necessary materials, resources and personnel in order to perform the Service.

With respect to non-standard Services classified as additional non-standard works, the following cost and task items required for the completion of the order have been specified:

| L.P. | SERWIS | POZYCJA ZADANIOWA | DESCRIPTION OF TASKS |
| --- | --- | --- | --- |
| 1. | Wire Line Services - Serwis Wire Line (W/L) | Mob/dem urządzenia W/L wraz z załogą (Baza – Mogilno / Mogilno – Baza) | Mob/Demob of W/L Unit and Personnel (Base - Mogilno / Mogilno – Base) |
| 2. |  | Mob/dem urządzenia W/L wraz z załogą (Baza – Kosakowo / Kosakowo – Baza) | Mob/Demob of W/L Unit and (Personnel Base-Kosakowo / Kosakowo Base) |
| 3. |  | Mob/dem załogi wraz oprzyrządowaniem do prac przy uzbrojeniu komory (Mogilno) | Mob/demob Personnel and Downhole Equipment in between jobs Mogilno |
| 4. |  | Mob/dem załogi wraz oprzyrządowaniem do prac przy uzbrojeniu komory (Kosakowo) | Mob/demob Personnel and Downhole Equipment in between jobs Kosakowo |
| 5. |  | Mob/dem urządzenia W/L wraz z załogą (Kosakowo - Mogilno / Mogilno – Kosakowo) | Mob/Demob of W/L Unit and Personnel Kosakowo - Mogilno/ Mogilno - Kosakowo |
| 6. |  | Urządzenie W/L – stawka dzienna oparta o 8-godzinny dzień pracy urządzenia | W/L Truck - Base Charge - based on 8 hrs per day |
| 7. |  | Urządzenie W/L – stawka za dodatkowe godziny pracy urządzenia | W/L Unit – additional hr |
| 8. |  | Śluza ciśnieniowa 9" – stawka dzienna zestawu śluzy | Pressure Control Equipment – 9”- per day |
| 9. |  | Śluza ciśnieniowa 7" – stawka dzienna zestawu śluzy | Pressure Control Equipment – 7”- per day |
| 10. |  | Nożyce do pracy W/L – stawka dzienna za prace | Heavy Duty Spring Jar Assembly - per day |
| 11. |  | Urządzenie W/L – stawka dzienna przestoju urządzenia | W/L Unit – Standby Charge(per day) |
| 12. |  | Kierownik urządzenia W/L – stawka za 8-godzinny dzień pracy | W/L Supervisor - based on 8 hrs per day |
| 13. |  | Kierownik urządzenia W/L – stawka za dodatkowe godziny pracy | W/L Supervisor - additional hr charge |
| 14. |  | Asystent kierownika urządzenia W/L – stawka oparta o 8-godzinny dzień pracy | W/L Assistant - based on 8 hrs per day |
| 15. |  | Asystent kierownika urządzenia W/L – stawka za dodatkowe godziny pracy | W/L Assistant - additional hr charge |
| 16. | Downhole Video Services / Serwis badania kamera wideo | Mob/dem urządzenia do wgłębnej inspekcji kamerą (zespół ludzi/ narzędzia, sprzęt) | Mob/demob of Downhole Video Unit (logging crew, tools, equipment) |
| 17. |  | Badanie kamerą wgłębną – stawka za 10-godzinny dzień pracy (zespół ludzi/ narzędzia, sprzęt) | Down Hole Video Survey on 10 hours basis (logging crew, tools, equipment) |
| 18. |  | Badanie kamerą wgłębną – dodatkowe godziny pracy (zespół ludzi / narzędzia, sprzęt) | Down Hole Video Survey – each additional hr (logging crew, tools, equipment) |
| 19. |  | Oprzyrządowanie kamery wgłębnej – opłata dzienna przy przestoju (zespół ludzi / narzędzia, sprzęt) | DHV Equipment – Standby Charge (logging crew, tools, equipment) per day |
| 20. | Pobór solanki / Sampling brine | Zestaw do poboru solanki | Brine sampler |

The Contractor shall define the methodology of repairs and a detailed schedule of works together with the repair evaluation criteria, and shall present them to the Contracting Entity for approval.

As part of additional works (repairs), the Contractor shall be required to prepare documentation (e.g. service/maintenance report), including in particular:

    - details of the facility / installation / unit,

    - time when the works/activities were carried out,

    - methodology used to carry out the works (equipment, method, other relevant information),

    - recommendations and guidelines,

    - and photographic documentation, if available (confirming the extent of damage, necessary replacement of parts and sub-assemblies) or other necessary information/data required to compile detailed documentation.

### Additional works – Repairs of the downhole safety valve

The deadline for completion of activities related to additional repair works (replacement of parts and sub-assemblies) of the downhole safety valve (unit) shall be agreed with the Contracting Entity, provided, however, that for service activities it shall not exceed 30 days from the date of Service Order (contingent on the supply of necessary parts and materials).

An appropriate order will be issued for these repair activities, specifying the scope of activities and the supply of parts and sub-assemblies for the purposes of the complete service of replacing parts and sub-assemblies of the downhole safety valve.

With respect to additional works classified as repairs, the following cost and task items required for the execution of the subject of the order have been specified:

| L.P. | POZYCJE ZADANIOWE – Prace dodatkowe | DESCRIPTION OF TASKS – additional tasks |
| --- | --- | --- |
| 1. | Wymiana uszczelnienia zamka zaworu 7,05” | Replacement of Packing Mandrel 7,05” valve |
| 2. | Wymiana uszczelnienia zamka zaworu 5,813” | Replacement of Packing Mandrel 5,813" valve |
| 3. | Wymiana korpusu zaworu 7,05” | Replacement of Collet 7,05” valve |
| 4. | Wymiana korpusu zaworu 5,813" | Replacement of Collet 5,813" valve |
| 5. | Wymiana tulei zaworu 7,05” | Replacement of Poppet Seatnickel plated 7,05” |
| 6. | Wymiana tulei zaworu 5,813" | Replacement of Poppet Seat nickel plated 5,813” |
| 7. | Wymiana standardowego kompletu części zamiennych w zaworze 7,05” | Replacement of Standard 7,05” Redress Kit per Valve |
| 8. | Wymiana standardowego kompletu części zamiennych w zaworze 5,813” | Replacement of Standard 5,813" Redress Kit per Valve |
| 9. | Wymiana szczęk posadowczych zaworu 7,05” | Replacement of keys 7,05” valve |
| 10. | Wymiana szczęk posadowczych zaworu 5,813" | Replacement of keys 5,813" valve |
| 11. | Regeneracja poprzez niklowanie zaworu grzybkowego 7,05” | Sleeve Poppet 7,05” valve nickel plating |
| 12. | Regeneracja poprzez niklowanie zaworu grzybkowego 5,813" | Sleeve Poppet 5,813" valve nickiel plating |
| 13. | Wymiana zaworu grzybkowego 7,05” | Replacement of Sleeve Poppet 7,05” valve |
| 14. | Wymiana zaworu grzybkowego 5,813" | Replacement of Sleeve Poppet 5,813" valve |
| 15. | Wymiana tulei zaworu grzybkowego 7,05” | Replacement of Poppet Seat 7,05” valve |
| 16. | Wymiana tulei zaworu grzybkowego 5,813" | Replacement of Poppet Seat 5,813" valve |
| 17. | Inspekcja i ocena wgłębnego zaworu bezpieczeństwa w specjalistycznym warsztacie producenta, celem przywrócenia sprawności technicznej (regeneracji) i ponownej zabudowy | Inspection and verification of a downhole safety valve in a manufacturer's workshop to restore technical efficiency (regeneration) and re-installation |

Parts for the execution of the specified additional works shall be supplied and secured by the Contractor.

The Contractor shall define a methodology of works with respect to the repair and/or replacement of parts and sub-assemblies of the unit, and other necessary activities required to restore full functionality of the unit.

As part of additional works (repairs), the Contractor shall be required to prepare documentation (e.g. service/maintenance report), including in particular:

    - details of the facility / installation / unit,

    - time when the works / activities were carried out,

    - methodology used to carry out the works (equipment, method, other relevant information),

    - recommendations and guidelines,

    - and photographic documentation, if available (confirming the extent of damage, necessary replacement of parts and sub-assemblies).

### Additional works – Assembly and disassembly of mechanical plugs in the well

Removable mechanical downhole plugs are to be installed in the 7.05" and 7.25" and 5.813" and 5.963" landing nipples or in the casing0, body.

As part of the service activities, the Contractor shall ensure all the necessary materials, resources and personnel to carry out the task. The works carried out in the scope of the planned activities related to the assembly and disassembly of mechanical plugs must guarantee full safety of their execution, specifically for site works at the CUGS undertaken in an active installation under the pressure of gas kept in storage.

In the event of circumstances preventing field works from being carried out within the agreed time, the Contractor shall be required to justify and indicate the reasons why the timely completion of the works is impossible.

An order specifying the scope of activities and deliveries for the purposes of a complete Service and additional works shall be issued for the activities in question.

The Contractor shall define a methodology of works / activities and deliveries with respect to the mechanical plug assembly / disassembly operations in the well, and other necessary activities required to restore full functionality of the unit (storage cavern).

The scope of works for the mechanical plug assembly / disassembly service in the gas storage cavern well shall include in particular the following activities:

#### Installation of mechanical plugs in the well

Performance of a service of setting removable mechanical downhole plugs in the landing nipples or in the pipe body – the expected execution time is about 2 days:

| ITEM | SPECIFIC DESCRIPTION - Installation of plugs in the well |
| --- | --- |
| 1. | Mobilize the logging unit and a service crew to the location where the Service is to be carried out (including, in particular, the provision of the Wire Line unit, pressure control equipment, drift, mechanical plug setting tool, mechanical plugs, drill collars, spring jar, crew for supervision and execution of the works). |
| 2. | Set up the slick line (S/L) unit with a spooled up steel wire, and the pressure control equipment, assembly top flange on wellhead. |
| 3. | Place the assembly with a gauge in the pressure control equipment. |
| 4. | Install the pressure control equipment on the cavern head. |
| 5. | Run the drift assembly and drift the casing to the landing depth of the mechanical plug at the lower landing nipple, pull the drift assembly. |
| 6. | Replace with an assembly with a tool and a mechanical plug. |
| 7. | Run and set mechanical plug at the lower landing nipple or a predefined setting depth in the pipe body. |
| 8. | Pressure test of the mechanical plug. |
| 9. | Run and set mechanical plug at the upper landing nipple or a predefined setting depth in the pipe body. |
| 10. | Disassembly the pressure control equipment and the Slick Line, disassembly top flange on wellhead. |
| 11. | Demobilize the unit together with the crew (return to base). |
| 12. | Preparation of post-completion documentation. |

Note: Prior to the installation of the safety plugs, the downhole safety valve in the cavern well will be removed.

Note: The expected time interval between setting and pulling out (disassembly of) the plugs in the well is about 14 calendar days.

#### Disassembly of mechanical plugs in the well

Performance of a service consisting in the disassembly and pulling up the removable mechanical downhole plugs – the  expected execution time is about 2 days:

| ITEM | DETAILED DESCRIPTION – Disassembly of mechanical plugs in the well |
| --- | --- |
| 1. | Mobilize the logging unit and a service crew to the location where the Service is to be carried out (including, in particular, the provision of the Wire Line unit, pressure control equipment, drift, release and pulling tool for the mechanical plug, drill collars, spring jar, crew for supervision and execution of the works). |
| 2. | Set up the wire line (W/L) unit with a spooled up steel wire, and the pressure control equipment, assembly top flange on wellhead. |
| 3. | Place a release and pulling assembly for the mechanical plug in the pressure control equipment. |
| 4. | Install the pressure control equipment on the cavern head. |
| 5. | Run the assembly with a release tool, release and pull the mechanical plug from the upper landing nipple, or from a predefined setting depth in the pipe body. |
| 6. | Run the assembly with a release tool, release and pull the mechanical plug from the lower landing nipple, or from a predefined setting depth in the pipe body. |
| 7. | Disassembly the pressure control equipment and the wireline unit, disassembly top flange on wellhead. |
| 8. | Demobilize the unit together with the crew (return to base). |
| 9. | Preparation of post-completion documentation. |

Note: After removing the retrievable mechanical downhole plugs, a downhole safety valve will be installed in the cavern well.

The scenario is considered where  retrievable mechanical downhole plugs would be set in the pipe body at a different time than the period resulting from the planned maintenance cycle for the downhole safety valves.

The above mentioned works should be carried out when the assembly is being run into the hole, with a single trip being allowed where the running rate must not exceed 30 meters per minute.

As part of additional works, the Contractor shall be required to prepare documentation (e.g. service/maintenance report), including in particular:

    - details of the facility / installation / unit,

    - time when the works/activities were carried out,

    - methodology used to carry out the works (equipment, method, other relevant information),

    - recommendations and guidelines,

    - and photographic documentation, if available (confirming the completed activities, other).

All costs of transporting the products to the indicated location of the Contracting Entity shall be borne by the Contractor (according to the DDP INCOTERMS formula).

In case of additional works, the following cost and task items necessary for the execution of the subject of the order have been specified:

| ITEM | POZYCJE ZADANIOWE – Prace dodatkowe | DESCRIPTION OF TASKS – additional tasks |
| --- | --- | --- |
| 1. | Prace montażu/demontażu korków w odwiercie | Assembly/Disassembly works for plugs in well |
| 2. | 7,05 RN Korek z łącznikiem wyrównawczym (minimum 1 dzień) | 7,05 RN Plug with Equalizing sub and cap, One trip System (Minimum Charge 1 Day) |
| 3. | 7,05 RN Korek z łącznikiem wyrównawczym - opłata za dodatkowe dni | 7,05 RN Plug with Equalizing sub and cap, One trip System Additional Daily Charge |
| 4. | Zestaw części zamiennych na korek 7,05 RN | Spare Part Kit per run/redress 7,05 RN Plug Assembl |
| 5. | 7,25 RN Korek z łącznikiem wyrównawczym (1 dzień) | 7,25 RN Plug with Equalizing sub and cap, One trip System (Minimum Charge 1 Day) |
| 6. | 7,25 RN Korek z łącznikiem wyrównawczym - opłata za dodatkowe dni | 7,25 RN Plug with Equalizing sub and cap, One trip System Additional Daily Charge |
| 7. | Zestaw części zamiennych na korek 7,25 RN | Consumables Spare Part Kit per run/redress 7,25 RN Plug Assembly |
| 8. | 5,813 RN Korek z łącznikiem wyrównawczym (minimum 1 dzień) | 5,813 RN Plug with Equalizing sub and cap, One trip System Minimum Charge (1 Day) |
| 9. | 5,813 RN Korek z łącznikiem wyrównawczym – opłata za dodatkowe dni | 5,813 RN Plug with Equalizing sub and cap, One trip System Additional Daily Charge |
| 10. | Zestaw części zamiennych na korek 5,813 RN | Consumables Spare Part Kit per run/redress 5,813 RN Plug Assembly |
| 11. | 5,963 RN Korek z łącznikiem wyrównawczym (1 dzień) | 5,963 RN Plug with Equalizing sub and cap, One trip System (Minimum Charge 1 Day) |
| 12. | 5,963 RN Korek z łącznikiem wyrównawczym – opłata za dodatkowe dni | 5,963 RN Plug with Equalizing sub and cap, One trip System Additional Daily Charge |
| 13. | Zestaw części zamiennych na korek 5,963 RN | Consumables Spare Part Kit per run/redress 5,963 RN Plug Assembly |
| 14. | Odpinalny korek wgłębny dedykowany do zapięcia w caliźnie rury 7” lub 8 5/8” (minimum 1 dzień) | Retrievable Bridge Plug dedicated to CSG 7” or 8 5/8’’ (Minimum Charge 1 Day) |
| 15. | Odpinalny korek wgłębny dedykowany do zapięcia w caliźnie rury 7” lub 8 5/8’’ – opłata za dodatkowe dni | Retrievable Bridge Plug dedicated to CSG 7” or 8 5/8’’ Additional Daily Charge |
| 16 | Opcjonalnie zapasowy odpinalny korek wgłębny dedykowany do zapięcia w caliźnie rury 7” lub 8 5/8”- opłata dzienna | Optional Back-Up RBP 7 or 8 5/8” daily charge |
| 17 | Urządzenie elektryczne do zapinania korków, narzędzie do zapięcia i wypięcia korka opłata za dzień | DPU, Setting Tool and Pulling Tool per day rental charge |
| 18 | Opcjonalnie zapasowe urządzenie elektryczne do zapinania korków, narzędzie do zapięcia i wypięcia korka opłata za dzień | Optional Back-up DPU, Setting Tool and Pulling Tool per day rental charge |
| 19 | Opcjonalnie szablon narzędzie do zbierania zanieczyszczeń / opłata za dzień | Optional Junk Basket per day rental charge |
| 20 | Zestaw części zamiennych na odpinalny korek wgłębny dedykowany do zapięcia w caliźnie rury 7” lub 8 5/8’’ | Spare Part Kit per run/redress Retrievable Bridge Plug dedicated to CSG 7” or 8 5/8’’ assy |

# METHOD AND TIME OF ORDER DELIVERY

All the identified requirements in terms of activities, materials or tools necessary for the completion of the inspection task shall rest with the Contractor (the Contractor shall be required to take into account all costs of services and materials, etc. Parts and materials / Travel and transport costs / Working time and mobilization / Accommodation costs / Costs of specialist equipment and tools, etc.).

The required deadline for the completion of each order shall be set in the individual Orders to be placed by the Contracting Entity, provided that it is not be shorter than 30 calendar days.

The service is planned to be performed at given locations in Q3-Q4 of the year 2026 and 2027, but it may be postponed due to the nature of the CUGS operations. The Contracting Entity does not guarantee the mobilisation and aggregation of the works during a single consecutive period.

The planned quantities and specified storage caverns planned during the term of the Agreement for basic activities in individual facilities are presented in the preliminary schedule in the table below.

TABLE - Scope of inspections and logging in production wells and storage caverns in the years 2026-2027

The schedule is only a preliminary arrangement and may be subject to change with regard to the indicated storage caverns – the Contracting Entity does not expect the number of declared basic inspections to be reduced.

The deadlines for the execution of site (field) works must be confirmed by the Contractor by notifying the persons and providing the completed Register of Authorizations – the information must be provided at least 48 hours before undertaking the works.

# OTHER MATERIAL CONDITIONS OF THE CONTRACT

Before undertaking any on-site activities, an organisational meeting between the Contracting Entity’s and the Contractor’s representatives shall be held to discuss the organisation and coordination of the works and to designate the responsible persons.

The division of responsibility with regard to the performance of the Subject of the Contract may be clarified in detail at the stage of preparing a detailed schedule of works.

In the event of the occurrence of circumstances necessitating the work during hours other than between 7:00 a.m. and 3:00 p.m. on business days, the Contractor shall inform the Contracting Entity about such need.

Each interruption of works on the site shall be reported by the Contractor to the Contracting Entity’s representative, together with information about the reasons for the interruption, and subject to ensuring the protection of workstations, tools and components of the equipment / facility handed over to the Contractor.

During the disassembly activities, the Contractor shall be required to protect the open parts of the infrastructure / equipment / installation so as to prevent any “foreign” components, parts, tools, etc. from penetrating therein, which could significantly damage or compromise proper operation of the equipment.

## OBLIGATIONS OF THE CONTRACTING ENTITY

For the purposes of providing Services at the site of CUGS facility, the Contracting Entity shall provide:

•	Staff amenities,

•	230/400 V power supply,

•	Crane service,

•	Cherry picker,

•	Access to the unit / installation / facility for the purposes of the performance of the Subject of the Contract. The Contracting Entity shall make the unit available during the period defined at the schedule definition stage, for a time not longer than specified in the order, on business days,

•	Dedicated measuring devices to monitor gas concentration levels within the explosion hazard zone,

•	Lockout tagout systems protecting the units from accidental start-up,

- Disconnection of cathodic protection from the storage cavern well,

- Disconnection of the 9”-5K ball valve’s drive.

The Contracting Entity undertakes to provide the Contractor with access to the units covered by the Subject of the Contract, at times and dates agreed with the Contracting Entity.

As part of the preparatory works, the Contracting Entity shall secure the equipment against power supply shut-down for the main power equipment installed in the facility which is subject to the operational tasks.

## SPECIFIC REQUIREMENTS FOR THE CONTRACTOR

The personnel of the Contractor and its subcontractors shall be required to comply with the organizational arrangements being in force in the Contracting Entity’s organisation as well as the health and safety, fire and environmental protection regulations.

The Contractor shall be required to comply with the Polish regulations on working time and occupational health and safety resulting from the restrictions applying on site, and specifically the Contractor shall protect the employees with the necessary personal protective equipment.

The Contractor shall demonstrate adequate authorizations and qualifications of the crews to be engaged for the performance of the works, and such qualifications must be valid and confirmed before any works are undertaken at the CUGS. The possession of the qualifications and permits shall be evidenced according to the information list and form provided by the Contracting Entity (Register of Authorisations).

Required qualifications for the Contractor’s personnel:

    - The person who is to exercise supervision over the works carried out on the site as the team leader (document confirming management qualifications),

    - Specialist for operating a wireline or electric cable unit (document confirming formal qualifications required to operate the wireline or logging unit),

    - A qualified operator of the pressure control equipment(document confirming formal qualifications required to operate pressure control equipment),

    - Specialist for performance of measurements with a downhole video camera (document confirming the qualifications to operate a downhole video camera),

The above team of the Contractor should have at least 5 years of experience in performing the work.

The Contractor’s personnel should have current certificates in respect of health and safety training and medical examinations.

Before undertaking the performance of any works, the Contractor shall obtain, through the person indicated in the Agreement, the consent of the Manager of the Underground Cavern Gas Storage Facility for the entry of vehicles and employees to the site of the underground gas storage where the work is to be carried out. For this purpose, no later than 3 business days before the commencement of the works, the Contractor shall be obliged to provide the Contracting Entity (i.e. the person indicated in the contract) with the following information in writing or by e-mail:

a) a list of persons with passport (ID card) numbers who will perform work on the premises of the underground gas storage facility, with indication of persons supervising the work,

b) a list of vehicles (make and registration number, driver’s details) and the equipment to be used during the works,

c) the completion date of the works (date and time of commencement and completion of the work),

d) telephone numbers of persons supervising the works,

e) the register of authorisations – according to the enclosed exhibit.

The Contractor shall be responsible for the disposal of materials and parts generated during the performance of the work.

### REQUIREMENTS CONCERNING SPECIALIST EQUIPMENT

The tools shall comply with all the restrictions which guarantee that the job/ operations are carried out in accordance with the procedures recommended by the equipment manufacturer and the good drilling practices.

The equipment and tools used shall comply with all the restrictions in respect of their operation, use and installation. Specialised equipment must guarantee, in particular, that the restrictions and requirements for maximum acceptable technical and quality parameters applied by the the Contracting Entity are conformed to.

At the request of the Contracting Entity, the Contractor shall submit specifications of the equipment, instruments, Slick Line Unit (S/L), Wire Line (W/L) unit, pressure control equipment and accessories which it uses or intends to use for the execution of the Subject of the Order, as well as certificates conforming the conformity with the requirements concerning technical and quality parameters specified below.

The Contracting Entity shall not be liable for the loss of equipment and tools during site works carried out in the as a result of improper handling by Contractor personnel.

#### Pressure control equipment

The pressure control equipment should comprise, in particular, the following components:

  - Flange connection – an element of the pressure control equipment to connect the upper part of the christmas tree with the pressure control equipment,

  - Blow-out preventer (BOP) – specialist device designed to seal the logging cable or wireline in case of unforeseen events such as damage and repair of the cable, problems with the pressure control equipment modules above the BOP (in particular the loss of tightness of the grease injection head feeding lubricant to the cable tubing) which ensure well pressure compensation and safe up/down movement,

  - Tool Trap – a specialist device which ensure controlled running and pulling of the logging unit. The dimension of a a gap in the trap is larger than the diameter of the cable and smaller than the diameter of the logging unit. After being retrieved from the well, the trap drops thereby locking the unit in the pressure control equipment (required for a downhole video camera),

  - Quick test sub – a specialist device for the controlled performance of the pressure test of the pressure control equipment (optional),

  - Lubricators section – a pipe section of appropriate length and diameter connected to the other components of the pressure control equipment to isolate equipment and tools in the controlled spaces,

  - Grease injection head – a specialist device ensuring the sealing of a cable / steel wire, well pressure compensation and maintaining the tightness of the pressure control equipment,

  - Degassing unit – a set of conduits ensuring controlled gas release outside the zone where the service works are carried out,

  - Measuring instrumentation – local measuring instruments of appropriate accuracy class and suitable for maximum permissible technical parameters (pressure, temperature),

  - Shut-off fittings – a system of valves and gate valves used to shut off certain space for the purposes of controlling the process of carrying out specific jobs / field (site) operations and works.

  - Pipe wrenches and a pneumatic wrench for assembly/disassembly of the wellhead flange.

TABLE – Essential technical and quality requirements for the pressure control equipment

DETAILED DESCRIPTION

- Working pressure for the 5000 psi (35 MPa) pressure control equipment – for a valve set on 7" and 8 5/8" casing, - Length of the lubricator sections to enable running of the downhole valve assembly together with drill collars, the downhole video camera, gauge, pulling / running tools to be used for the downhole safety valve assembly (taking into account the length and diameter), - BOP for steel wire sealing, - BOP electrical/fibre optic cable sealing, - Flange connection 7 1/16” -3K, - Flange connection 9” – 5K, 9” – 3K, - Operation in natural gas environment, - Shut-off fittings and control and instrumentation systems (local), - Unit inspection log – for the pressure control equipment according to the maintenance schedule – available for inspection by the Contracting Entity.

#### Downhole video camera

TABLE – Essential technical and quality requirements for the downhole video camera unit

DETAILED DESCRIPTION

- Working pressure of at least 5000 psi (35 MPa), - Operating temperature above 70°C, - Operation in natural gas environment, - Digital video recording, - Instant preview function, - Side view camera (optional mode), - Possibility of generating photos from the image recording at a specific depth, - Another backup downhole video camera set available on location.

The tool has to conform to all the requirements in respect of the inspection and assessment of technical condition, as described for the visual inspection of the well and cavern neck (according to section 3.1.1.2 of the DDoSoC).

#### Service tool kits for downhole safety valves

Table – Essential technical requirements for the tool kits to be used in downhole safety valve operations:

DETAILED DESCRIPTION

- Working pressure of at least 5000 psi (35 MPa), - Operating temperature above 70°C, - Operation in natural gas environment, - Sinker bars suitable for the expected wellhead pressure to run a gauge assembly, run and pull up the running and pulling assemblies, - Heavy duty spring jar for running and pulling up a downhole safety valve, - Dedicated tools for removing the downhole safety valve from the landing nipple, its pulling up, re-running and re-setting in the landing nipple, - For all tools, spring jar, sinker bars and parts of the kit not listed above a backup should be available on site.

The tool kits for pulling up / running downhole safety valves should match the equipment manufacturer’s types specified in the table – Specification of downhole safety valves (section 2.2. of the DDoSoC).

#### Wireline logging unit

TABLE – Essential technical and quality requirements for the Wire Line WL logging unit/Slick Line SL:

DETAILED DESCRIPTION

- Logging unit (not older than 10 years), - Two independent power supply sources for the logging unit (main power supply and power generator), - Certified cable reels, - Steel wire / electric (logging) cable not older than 4 years – together with specification, - Minimum length of the steel wire / logging cable – at least 1500 m, - Required logging rate from 1 m/min to 60 m/min, - Efficient system tracing the depth and tension of the cable, - Chains, slings, ropes, shackles with valid certificates.

NOTE: In the event of malfunction of the logging equipment or downhole camera units, the Contractor shall mobilise, at own expense, additional equipment enabling proper performance of the task within 24 hours of the call.

# APPENDIX

Appendix 1.1.A. Specification of storage cavern wells of MOGILNO CUGS

Appendix 1.1.B. Specification of storage cavern wells of KOSAKOWO CUGS

Appendix 1.2. Christmas tree diagram

Appendix 1.3. Work order for hazardous work in explosive gas atmosphere

Appendix 1.4. Work order for particularly hazardous

Appendix 1.5. Well handover certificate

Appendix 1.6. Works acceptance report

Appendix 1.7. Register of authorizations

Appendix 1.8. Downhole safety valve diagram

|  | Specification of wells/storage caverns. Technical data of the wells / caverns of CUGS Kosakowo Annex No. 1.1.B to the DDoSoC |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Item |  | Unit | K-1 | K-2 | K-3 | K-4 | K-5 | K-6 | K-7 | K-8 | K-9 | K-10 |
| casing | 13 3/8” section |  | [m bgl] | 147.0 | 171.0 | 170.0 | 171.0 | 171.0 | 175.0 | 175.0 | 174.0 | 175.0 | 175.0 |
|  | 9 5/8” section |  | [m bgl] | 1020.0 | 1021.6 (1069*) | 1019.0 (1075*) | 1020.2 (1073*) | 1019.7 (1068*) | 1020.0 | 1020 (1066*) | 1020 (1077*) | 1017 (1059*) | 1022 (1101*) |
|  | 7” section |  | [m bgl] | 992.5 | 984.3 | 983.3 | 989.6 | 989.4 | 983.8 | 981.3 | 982.9 | 981.6 | 986.5 |
|  | Fracture gradient |  | [kPa/m] | 21.5 | 21.5 | 21.5 | 21.5 | 21.5 | 21.5 | 21.5 | 21.5 | 21.5 | 21.5 |
| HALLIBURTON downhole equipment | Latch type |  | [-] | Ratch-Latch |  |  |  |  |  |  |  |  |  |
|  | Production packer TWR 8 5/8'' |  | [m bgl] | 994.8 | 986.1 | 985.5 | 991.3 | 992.4 | 986.2 | 985.1 | 986.1 | 984.8 | 990.3 |
|  |  |  | [psi] | 8,000 | 8,000 | 8,000 | 8,000 | 8,000 | 8,000 | 8,000 | 8,000 | 8,000 | 8,000 |
|  | Landing nipple 5,813" RPT 7500psi |  | [m bgl] | 998.0 | 989.5 | 988.9 | 995.1 | 996.8 | 991.5 | 989.0 | 990.6 | 989.3 | 994.2 |
|  | Landing nipple 5,813" RNT 7500 psi |  | [m bgl] | 1001.3 | 992.7 | 992.2 | 998.3 | 1000.2 | 994.9 | 992.7 | 994.0 | 992.7 | 997.9 |
|  | Production casing shoe |  | [m bgl] | 1005.0 | 995.8 | 995.3 | 1001.4 | 1003.3 | 998.1 | 996.5 | 997.2 | 995.9 | 1001.7 |
| cavern geometry | Cavern top |  | [m bgl] | 1040.00 | 1030.00 | 1037.00 | 1037.50 | 1035.00 | 1033.0 | 1038.0 | 1036.0 | 1032.5 | 1034.5 |
|  | Gas-brine boundary level |  | [m bgl] | 1124.90 | 1119.00 | 1139.80 | 1143.50 | 1114.70 | 1128.0 | 1121.6 | 1121.6 | 1135.8 | 1122.8 |
|  | Cavern base – axis/year of measurement |  | [m bgl] | 1126.80 | 1124.80 | 1145.00 | 1145.40 | 1119.00 | 1132.0 | 1125.0 | 1124.3 | 1038.4 | 1126.0 |
| operating parameters | Max. wellhead pressure |  | [MPa] | 16.20 | 16.20 | 16.20 | 16.20 | 16.20 | 16.20 | 16.00 | 16.20 | 16.00 | 16.00 |
|  | Min. wellhead pressure |  | [MPa] | 3.70 | 3.70 | 3.70 | 3.70 | 3.70 | 3.70 | 3.70 | 3.70 | 3.70 | 3.70 |
| Wellhead | Type of connection |  | [-] | 7 1/16" - 3K |  |  |  |  |  |  |  |  |  |
| * - MD |  |  |  |  |  |  |  |  |  |  |  |  |  |

Appendix 1.3 to the DDoSoC

…………………

Stamp of the unit in charge of the execution of works

Work order for hazardous work in explosive gas atmosphere No.    ....

dated     ………….

      - Date and time of beginning the works:               [date], [time]

      - Date and expected time of completion of the works:                [date], [time]

      - Brief description of the work in explosive gas atmosphere, indication how gas flow is to be shut off

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………….………………………………………………………………………………

      - Indication of the location of work:             …………………………………

      - List of crew members by name (to be completed on the job day):

………………..…………

………………..…………

………….........…………

      - Person responsible for completion of the work:

…………………………

      - List of safety equipment:

……………………………

……………………………

      - Training on the work to be carried out were provided by:

(name and surname)         …………………  ………

- Issued by:          ……………………….

(stamp-name and signature)

- The order was agreed with:

……………………………………………………………………………………

- The order for non-typical work was consulted upon in respect of process considerations

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

The start and completion of works to be reported to the dispatcher at the Mogilno/Kosakowo CUGS.

(Stamp – signature)

- Approved by

(Stamp – signature)

- Instructions for the above mentioned works

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………....

REPORT ON THE COMPLETION OF WORKS:

- Completed on [date, time]

- Full scope of works completed– yes/no

……………………………………………………………………………………………………………………...…

The completion of works reported to (surname and first name of the dispatcher)

Signature of person responsible for supervision	Signature responsible 
		for carrying out the work.

Appendix 1.4 to the DDoSoC

Work order for particularly

hazardous work No. ...... dated ..............

- Location

…………………………………………….

- Type of particularly hazardous work

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

- Starting date and time:

………………………………………….

- Expected completion date and time

…………………………………………

- Crew member names (to be filled in on the day when the work is performed):

1.……………………………………………………                            2..………………………………………………..

3…………………………………………………….                            4………………………………………………….

5…………………………………………………….                            6………………………………………………….

7…………………………………………………….

- Supervisor of works*:

- for the Contracting Entity

……………………………

Signature ……………………………………………………………….

- for the Contractor

…………………………..

Signature ………………………………………………………………..

- Coordinator of works (in case of simultaneous execution of interdependent particularly hazardous work by different teams) - not applicable

Responsibilities of the coordinator and how they are carried out - not applicable

- List of safety equipment:

    - Necessary personal protective equipment

- Protective helmet

- Safety glasses

- Mask

- Protective gloves

- Protective anti-electrostatic workwear

- Harnesses and safety lines

- Ear muffs

- Work boots with a protective toecap

    - Necessary fire-fighting equipment (hand-held extinguishing equipment)

- fire blanket

- fire extinguisher

- Instruments for methane detection and measurement of oxygen concentration

- Explosimeter

- Gas content meter

- Means of communication

- Radiotelephone

- Telephone

e) First aid equipment (kept at the dispatcher's)

- Mobile first aid kit

- Stretcher

- Instruction on the work was given by ...................................................................

(Name and surname)

- Work order issued by …………………………………………………………………………………………

(stamp and signature)

- Work order approved by

(stamp and signature)

- I have read the instructions for carrying out the above mentioned work

  - …………………………………..

  - ………………………………….

  - ………………………………….

  - …………………………………..

WORK COMPLETION REPORT:

- Works completed, date, time .................................................................................................................

- Full scope of works completed yes/no
…….……………………………………………………………………

- Notification of completion of works (legible signature of dispatcher)
...............................................................

- Signature of person responsible for supervision and execution of works

- for the contracting entity			b) for the contractor *

………………………………………………………………..                                   ..…..………………………………………………………..

*- In the case when no third-party companies are involved, i.e. the work is performed exclusively by the employees of GSP Sp. z o.o., subsection b) is not applicable.

APPENDIX 1.5. DDoSoC

CUGS	,date:

Handover document confirming the delivery of well no. ....

for performance of specialised work in wells/storage caverns

ENTREPRENEUR (OPERATOR):

GAS STORAGE POLAND Sp. z o.o.; ul. Rumska 28, 81-198 Dębogórze, Poland

ENTITY / CONTRACTOR:

BASIS FOR PERFORMANCE OF THE WORKS (contract/order no.):	dated:

1)	DESIGNATION OF THE WELL/STORAGE CAVERN:

Well/cavern number:		Date of work (from/to):

Locality:		Voivodeship:

2)	TECHNICAL CONDITION OF THE WELL/STORAGE CAVERN:

Casing depth:	[m bgl],	Pipe diameter:	[inch]

Well (cavern depth):	[m bgl]

Wellhead type:

Wellhead fluid pressure:	[MPa]	Fluid type:

Equipment earthing method:	 Measurement by ohmmeter: YES / NO

3)	TYPE OF SPECIALISED WORK:

| - Well drifting: YES / NO | Drift diameter: [mm] |
| --- | --- |
| - Well logging - YES / NO* | Type of log: |
| - Geometry and volume logging - YES / NO* | Type of log: |
| - Removal / installation of downhole safety valve - YES / NO* |  |
| - Check of the technical condition of the well – DHV - YES / NO* |  |
| - Installation/removal of mechanical downhole plugs in the well - YES / NO* |  |
| - Logging of the well/cavern base - YES / NO* |  |
| - Other: |  |

4)	TYPE OF EQUIPMENT AND TOOLS FOR SPECIALISED WORK:

- Logging equipment::

- Specialist tools:

- Cable type:	Logging interval: from	[m] to	[m]

5)	IDENTIFIED RISKS OF FAILURE DUE TO TECHNICAL CONDITIONS OF THE WELL (tight spots, curvatures, etc.):

- OTHER HAZARDS RESULTING FROM THE DISTANCE FROM TECHNICAL INFRASTRUCTURE FACILITIES:

from a HV line: ………………………., radio and television transmitter: ………………………….

- DECLARATIONS, DELIVERY OF THE FACILITY FOR SPECIALIST WORKS:

7.1.1.	The delivering party declares that the well/cavern has been prepared for specialist works in accordance with the applicable regulations and does not pose a failure risk, and that the Contractor is authorised to carry out site works.

7.2. The accepting party declares that the equipment and tools to be run into the well have been checked for technical performance and do not pose a failure risk.

The devices / tools and equipment to be run into the well have been provided with adequate measurement instrumentation.

Delivered by:………………….….				Accepted by:……………………

(date, signature)……………………..				(date, signature):…………………..

- COMPLICATIONS DURING SPECIALIST WORKS:

..............................................................................................................................................................

..............................................................................................................................................................

- DECLARATION, DELIVERY OF THE FACILITY ACTER COMPLETION OF SPECIALIST WORKS:

8.1.The delivering party declares that the well/cavern  has been commissioned following the performance of specialist works in accordance with the applicable regulations and does not pose a failure risk and is fit for operation.

8.2. The accepting party declares that the technical condition of the well is known to it.

Delivered by:………………….….				Accepted by:……………………

(date, signature)……………………..				(date, signature):…………………..

Appendix 1.6 to DDoSoC

Appendix 1.7 to DDoSoC

| Register of authorisations of subcontractors engaged in the operation of the Mining Plant of Mogilno CUGS/Kosakowo CUGS*. |
| --- |
| Date | Company | No. of order/ contract under which the work is performed | Name and Surname | Work Activity | OHS training (date of training) | Medical examination (Validity date) | Other Authorisations |  | Signature of GSP’s verifier |  |  |
|  |  |  |  |  |  |  | Type of authorisation | No. |  |  |  |
| Personal data are processed exclusively for the purposes of contract performance and fulfilment legal requirements incumbent on the Data Controller. For more details about the obligation to provide information see https://ipi.gasstoragepoland.pl/wp-content/uploads/2018/04/Obowiazek-informacyjny-realizacja-umowy-z-kontrahentem.pdf |  |  |  |  |  |  |  |  |  |  |

Appendix 1.8 to DDoSoC