Białystok, 25.05.2022

Enclosure no. 1 Specification of the Object of Contract

Specification of the Object of Contract for the Request for Quotation no. 18/05/2022/3.2.1 PO IR

with reference to the implementation of the project no. POIR.03.02.01-20-0003/17 entitled „Implementation of the R&D Findings Leading to the Production of unique shrink sleeve labels with innovative perforation and effective convex and tactile printing ” under Measure 3.2 „Support for R&D results implementation”, and Sub-measure 3.2.1 “Research for the market” of the Intelligent Development Operational Programme, 2014-2020

Contract for co-financing no. POIR.03.02.01-20-0003/17-00 dated 30/11/2017r.

| No. | Object of the procurement | Technical specification/ minimum parameters |
| --- | --- | --- |
| 1. | The purchase and assembly of printing machine with specialized convex printing unit as a part of Shrink Sleeve Label (SSL) with convex printing – 1 pcs. | Minimum technical parameters and functionalities of printing machine with specialized convex printing unit as a part of Shrink Sleeve Label (SSL) with convex printing – 1 pcs 1; GENERAL INFO. 1,01; Press with flexible configuration that enables technological trials using simultaneous printing with flexo and rotogravure methods interchangeably on chosen printing stations (according to attached scheme).. 1,02; Printing on plastic films PE, PP, PVC, PET, PLA, OPS with thickness range: - PET 12um - 100um - PE, PP 20um - 100um - PVC 35um - 60um - PLA, OPS 40um - 60um. 1,03; Printing press enables printing web width at least 650 mm. 1,04; Printing speed 5 -150 m/min. 2; UNWINDER enables:. 2,01; Unwinding of reel with maksimum diameter 1m (1000 mm). 2,02; Unwinding of reels on sleeves d76 mm and d152 mm (internal diameter). 2,03; Reel loading on press from floor level. 2,04; Setting and reading rewinder parameters on main touchscreen panel or in surroundings of main touchscreen (to let human to operate press from one place). 2,05; Detector of the reel end. The measurement of the diameter of the unwound roll shall not be affected by lighting conditions and shall provide a measurement accuracy of +/- 3mm.. 2,06; Detection of web cracking. Quick machine stop in the situation with cracking web.. 2,07; Controlling the unwinding tensions by a servomotor. 3; WEB CLEANING. 3,01; Solution enables cleaning of two sides of web without contact.. 4; CORONA TREATMENT. 4,01; Enables treating of two sides of web. 4,02; Solutions enables to set and to read corona parameters on main touchscreen panel or on direct surroundings of main touchscreen, to let human to operate from one place. 5; FLEXO PRINTING UNIT. 5,01; Press enables printing simultaneously with 12 flexographic units with curing/drying systems acc. to the machine scheme. 5,02; Press enables curing/ drying on flexographic unit with UV arc lamp light or interchangeably UV LED radiation or interchangeably hot air acc. to the machine scheme. 5,03; Press is equipped in 3 sets of plate cylinders for fixing polymer flexo plates with three different printing repeatability within the range offered by the machine design.. 5,04; Open doctor-blade system for each flexographic unit. 5,05; Addidional interchangeable enclosed 2 chambers doctor-blade system for a) flexographic unit b) hot air unit. 5,06; Enables ink circulation on the each printing unit or other equivalent solution. 5,07; Enables adjusting pressure anilox - printing plate and printing plate - substrate. 5,08; Enables adjusting doctor blade pressure if applicable in unit design. 5,09; Printing unit uses 0,5 mm mounting tapes and 1,14 mm thickness printing plates. 5,10; Press is equipped in 22 ceramic anilox rollers with selected engraving parameters.Engraving parameters are in the range: volume from 1,5 cm3/m2 up to 40 cm3/m2; liniature from 25 l/cm up to 600 l/cm.. 5,11; Press is equipped in 12 additional (back up) open doctor-blade inktrays. 5,12; Press is equipped in additional tooling or equipment for printing unit changeovers, if applicable in press design. 6; ROTOGRAVURE PRINTING UNITS. 6,01; Press enables printing simultaneously with 3 rotagravure units and at least 8 flexo units according to the scheme of machine. 6,02; Press enables solutions to keep ink and varnish viscosity on 5 press units (3 roto, 2 flexo) including paint tanks with a maximum capacity max 15 l. 6,03; Each rotogravure unit enables ink circulation on the printing unit or other equivalent solution. 6,04; Cart for offline set up for each rotogravure unit. 6,05; Press enables in 3 additional chilling web cylinders after last rotagravure unit according to the scheme of machine. 6,06; Machine is equipped with electrostatic print assist system (ESA) on each rotogravure unit. 6,07; Press is equipped in 6 pressure cylinders with hardness of 85ShA: - 3 pressure delivered as a part of 2 units - 3 delivered as additional equipment. 6,08; Press is equipped with 3 additional sets of printing sleeve shaft bearings. 6,09; Enables to read and to set drying parameters on main touchscreen panel or on surroundings of main touchscreen. 7; TURN BAR OR EQUIVALENT SOLUTION. 7,01; The construction of the machine enables printing on both sides of the web by using a turn bar. Possibility of flexible positioning of the turn bar in accordance with the attached scheme: at least after printing units U1, U2, U13, U15.. 7,02; Press is equipped in additional tooling needed to operate and to assembly "turning web" solution on press if applicable in press design. 7,03; Press is equipped with microporous turnbar. The solution can't be based on a sleeve with holes.. 8; LAMINATOR, COLD FOIL UV. 8,01; The machine is equipped with one head enabling cold stamping and lamination on all flexographic units according to the presented machine scheme. 8,02; Diameter of reels wound from 400-600mm. Inner sleeve diameter: Internal diameter of reels: 76 mm. 8,03; The design of the laminator allows the adhesive transfer using a photopolymer plate and curing with each of the UV and UV LED drying systems. 8,04; Press is equipped in additional tooling needed to operate or to assembly lamination unit on press, if applicable in press design. 8,05; Enables application of several cold foil webs with min 50 mm width of each reel, unwinder and rewinder of the lamination unit equipped with friction clutch. 8,06; The station is equipped with 3 rubber pressing rollers with a hardness in the range of 85-90 ShA.. 8,07; Unit equipped with additional steel anvill roller with bearings (if this solution is applicable in design) as spare solution.. 9; UV CURING SYSTEM. 9,01; Enables to cure by UV arc lamp or UV LED bar on each flexo unit simultaneously or each type of flexo drying according to the machine configuration map and subsequent requirements. 9,02; Enables to cure by two UV arc lamps on the U1 unit.. 9,03; Enables to cure by dual UV arc and UV LED on F11-FL12 units.. 9,04; Enables to provide printing with speed 150 m/min with 100% of curing of white uv low migration ink with anilox 10 cm3/m2 by double UV ARC lamp and matt varnish low migration with anilox 10 cm3/m2 by double UV ARC lamp.. 9,05; Printer equipped with 14 UV cassetes (with UV bulbs).. 9,06; Minimum power of UV arc lamp is 180 W/cm. 9,07; Press is equipped in 6 UV LED bars, interchangeable with UV lamps. 9,08; Wave length for UV LED system: 395 nm, the power of bars at least 20 W/cm2. 9,09; Enables to cooperate with cooling installation in the customer's site, medium: glycol, 15 °C +/- 5 °C. 9,10; Enables to provide printing with speed at least 150 m/min.. 9,11; Enables to set and to read UV system parameters on press main touchscreen panel or on surroundings of main touchscreen, to let human to operate press from one place. 10; HOT AIR CURING SYSTEM. 10,01; Press is equipped with 4 hot air drying tunnels powered by electricity and enabling their simultaneous operation in the configuration shown in the scheme of machine. 10,02; Drying temperature is in the range 50°C - 90°C on each tunnels. 10,03; Each tunnel enables drying of water-based ink applied minimum with anilox 10 cm3/m2 (liniature 160 l/cm) and maksimum with rotogravure cylinder 48/0/130 l/cm, printing speed min 80m/min. 10,04; Drying system enables reproducible and ergonomic setting of air flow and temperatures. 10,05; Enables to read and to set drying parameters on main touchscreen panel or on surroundings of main touchscreen, to let human to operate press from one place. 11; AUTOMATIC REGISTER SYSTEM enables:. 11,01; Continuous control and correction of the fit during machine operation over the entire speed range. 11,02; Registration accuracy – max. 0,15 mm in 150 m/min during print run with all printing flexo stations and 80 m/min using three rotogravure units and the remaining flexo units. 11,03; Printing with barrier bands width max 25 mm (total). Including automatic register control system in both machine and transverse direction. 11,04; Registering when setting all units to work at a maximum of 50% of the web length in the machine. 11,05; Visibility of matt varnish application printed on 1st flexographic unit to enable the setting of registration. 11,06; Operation over the entire width of the web without additional setup operations. 12; DE-IONIZING WEB SOLUTION. 12,01; A solution for removing electrostatic loads by blowing ionized air in front of the rewinder powered by copressed air. 13; REWINDER enables:. 13,01; Rewinding of reel with diameter - at least 1000 mm. 13,02; Rewinding of reels on sleeves d76 mm and d152 mm (internal diameter). 13,03; Setting and reading rewinder parameters on main touchscreen panel or in surroundings of main touchscreen (to let human to operate press from one place). 13,04; Unloading the roll to the floor level. 13,05; Winding web pressure, with the web pressure force to be adjustable in a measurable and repeatable manner. 14; 'DIGITAL FEATURES enable. 14,01; Job recipe solution to save printing parameters. 14,02; Access to loop of 10 day history of machine diagnostic / maintenance alarms and errors. Data to transfer via communication port. Storage of machine operating parameters and the course of the process on the Masterpress server. 14,03; Extended maintenance information to read via main touchscreen panel (or other equivalent solution) and to contain: 1. Service alarm definition (f.e. ERROR 453) 2. Description what is suggested root cause of problem 3. What elements, part of machine need to be checked or diagnosed. 14,04; Monthly summary containing reports from the uv system including data on printing time, idle time, downtime and their % proportions. 15; VISUAL INSPECTION SYSTEM enables below features:. 15,01; Printing control within the entire print area (with x printing circumference) and different artworks on one photopolymer. 15,02; Control of: print registration, *bar code verification, *verification of master image to PDF file, * label dimensions, * printing length dimension, *stability. 15,03; Work with above values in maximum press speed. 15,04; Adjusting the illumination power of the measurement. 15,05; Work flow to transfer of information from printing machine to finishing machine including required equipment and components. 15,06; The cooperation of the press with the system: - marking sections with defects on the printed web - automatically stopping the machines in the subsequent stages of the process in order to remove the defective material. - equipped with appropriate devices and mechanisms supporting the above mentioned requirements.. 16; CONVEX PRINTING SYSTEM. 16,01; The equipment of the machine has to ensure printing of elements with a thickness of at least 0.22 mm by inkjet method, with transparent varnish. 16,02; The system has to be able to print at least 2 lines of inscriptions and warning signs. 16,03; The machine is to be equipped with additional devices and systems to enable power supply, control, communication, data management and printing of Braille and warning signs.. 17; GENERAL REQUIREMENTS. 17,01; Installation and training of operators in Masterpress factory in the operation of the machine and auxiliary systems, daily maintenance, problem solving. 17,02; Training of the maintenance team at the Masterpress factory on potential failures, maintenance, problem solving, establishing a list of items required to secure continuous operation of the machine. 17,03; Training the IT team on the requirements and operation of the communication systems installed on the press. 17,04; DDP (Delivered Duty Paid) Białystok - the organization and costs of loading and transport to the headquarters of the commissioning party at J. Kuronia 4 in Bialystok, insurance and possible customs duties are borne by the Supplier (the Seller) along with the risk associated with the loss or damage of the goods.. Machine must be brand-new. Tenderer is obliged to attach to the Tender Form a detailed specification of the contract object, which includes technical parameters and functionality of the device. The specification should contain descriptions relating to the functionality desirable in Enclosure no 1. |
|  | The purchase and assembly of printing machine with die-cutting and embossing unit as a part of Shrink Sleeve Label (SSL) with perforation production line – 1 pcs | Minimum technical parameters and functionalities of printing machine with die-cutting and embossing unit as a part of Shrink Sleeve Label (SSL) with perforation production line – 1 pcs, GENERAL. 1,01; Press with flexible configuration that enables technological trials using simultaneous printing with flexo and rotogravure methods interchangeably on chosen printing stations (according to attached scheme).. 1,02; Printing on plastic films PE, PP, PVC, PET, PLA, OPS with thickness range: - PET 12um - 100um - PE, PP 20um - 100um - PVC 35um - 60um - PLA, OPS 40um - 60um. 1,03; Printing press enables printing web width at least 650mm. 1,04; Printing speed 5 -150 m/min. 2; UNWINDER enables:. 2,01; Unwinding of reel with maksimum diameter 1m (1000 mm). 2,02; Unwinding of reels on sleeves d76 mm and d152 mm (internal diameter). 2,03; Reel loading on press from floor level. 2,04; Setting and reading rewinder parameters on main touchscreen panel or in surroundings of main touchscreen (to let human to operate press from one place). 2,05; Detector of the reel end.The measurement of the diameter of the unwound roll shall not be affected by lighting conditions and shall provide a measurement accuracy of +/- 3mm.. 2,06; Detection of web cracking. Quick machine stop in the situation with cracking web.. 2,07; Controlling the unwinding tensions by a servomotor. 3; WEB CLEANING. 3,01; Solution enables cleaning of two sides of web without contact.. 4; CORONA TREATMENT. 4,01; Enables treating of two sides of web. 4,02; Solutions enables to set and to read corona parameters on main touchscreen panel or on direct surroundings of main touchscreen, to let human to operate from one place. 5; FLEXO PRINTING UNIT. 5,01; Press enables printing simultaneously with 10 flexographic units with curing/drying systems acc. to the machine scheme. 5,02; Press enables curing/ drying on flexographic unit with UV arc lamp light or interchangeably UV LED radiation or interchangeably hot air acc. to the machine scheme. 5,03; Press is equipped in 3 sets of plate cylinders for fixing polymer flexo plates with three different printing repeatability within the range offered by the machine design.. 5,04; Open doctor-blade system for each flexographic unit. 5,05; Addidional interchangeable enclosed 2 chambers doctor-blade system for a) flexographic unit b) hot air unit. 5,06; Enables ink circulation on the each printing unit or other equivalent solution. 5,07; Enables adjusting pressure anilox - printing plate and printing plate - substrate. 5,08; Enables adjusting doctor blade pressure if applicable in unit design. 5,09; Printing unit uses 0,5 mm mounting tapes and 1,14 mm thickness printing plates. 5,10; Press is equipped in 20 ceramic anilox rollers with selected engraving parameters.Engraving parameters are in the range: volume from 1,5 cm3/m2 up to 40 cm3/m2; liniature from 25 l/cm up to 600 l/cm.. 5,11; Press is equipped in 10 additional (back up) open doctor-blade inktrays. 5,12; Press is equipped in additional tooling or equipment for printing unit changeovers, if applicable in press design. 6; ROTOGRAVURE PRINTING UNITS. 6,01; Press enables printing simultaneously with 2 rotagravure units and at least 9 flexo units according to the scheme of machine. 6,02; Press enables solutions to keep ink and varnish viscosity on 3 press units (2 roto, 1 flexo) including paint tanks with a maximum capacity max 15 l. 6,03; 'Each rotogravure unit enables ink circulation on the printing unit or other equivalent solution. 6,04; 'Cart for offline set up for each rotogravure unit. 6,05; 'Press enables in 2 additional chilling web cylinders after last rotagravure unit according to the scheme of machine. 6,06; 'Machine is equipped with electrostatic print assist system (ESA) on each rotogravure unit. 6,07; 'Press is equipped in 4 pressure cylinders with hardness of 85ShA: - 2 pressure delivered as a part of 2 units - 2 delivered as additional equipment. 6,08; Press is equipped with 2 additional sets of printing sleeve shaft bearings. 6,09; Enables to read and to set drying parameters on main touchscreen panel or on surroundings of main touchscreen. 7; 'TURN BAR OR EQUIVALENT SOLUTION. 7,01; The construction of the machine enables printing on both sides of the web by using a turn bar. Possibility of flexible positioning of the turn bar in accordance with the attached scheme: at least after printing units U1, U8. 7,02; Press is equipped in additional tooling needed to operate and to assembly "turning web" solution on press if applicable in press design. 7,03; Press is equipped with microporous turnbar. The solution can't be based on a sleeve with holes.. 7,04; Press is equipped in 2 turn bars. 8; LAMINATOR, COLD FOIL UV. 8,01; The machine is equipped with one head enabling cold stamping and lamination on all flexographic units according to the presented machine scheme. 8,02; Diameter of reels wound from 400-600mm. Inner sleeve diameter: Internal diameter of reels: 76 mm. 8,03; The design of the laminator allows the adhesive transfer using a photopolymer plate and curing with each of the UV and UV LED drying systems. 8,04; Press is equipped in additional tooling needed to operate or to assembly lamination unit on press, if applicable in press design. 8,05; Enables application of several cold foil webs with min 50 mm width of each reel, unwinder and rewinder of the lamination unit equipped with friction clutch. 8,06; The station is equipped with 3 rubber pressing rollers with a hardness in the range of 85-90 ShA.. 8,07; Unit equipped with additional steel anvill roller with bearings (if this solution is applicable in design) as as spare solution.. 8,08; The trolley for loading a roll of material on the unwinder of the laminating station. 9; UV CURING SYSTEM. 9,01; Enables to cure by UV arc lamp or UV LED bar on each flexo unit simultaneously or each type of flexo drying according to the machine configuration map and subsequent requirements. 9,02; Enables to cure by two UV arc lamps on the U1 unit.. 9,03; Enables to cure by UV and UV LED on FL8-FL10 unit in the configurations indicated in the scheme.. 9,04; Enables to provide printing with speed 150 m/min with 100% of curing of white uv low migration ink with anilox 10 cm3/m2 by double UV ARC lamp and matt varnish low migration with anilox 10 cm3/m2 by double UV ARC lamp.. 9,05; Printer equipped at least with 13 UV cassetes (with UV bulbs).. 9,06; Minimum power of UV arc lamp is 180 W/cm. 9,07; Press is equipped in 6 UV LED bars, interchangeable with UV lamps. 9,08; Wave length for UV LED system: 395 nm, the power of bars at least 20 W/cm2. 9,09; Enables to cooperate with cooling installation in the customer's site, medium: glycol, 15 °C +/- 5 °C. 9,10; Enables to provide printing with speed at least 150 m/min. 9,11; Enables to set and to read UV system parameters on press main touchscreen panel or on surroundings of main touchscreen, to let human to operate press from one place. 10; 'HOT AIR CURING SYSTEM. 10,01; Press is equipped with 4 hot air drying tunnels powered by electricity and enabling their simultaneous operation in the configuration shown in the scheme of machine. 10,02; Drying temperature is in the range 50 ºC - 90 ºC on each tunnels. 10,03; Each tunnel enables drying of water-based ink applied minimum with anilox 10 cm3/m2 (liniature 160 l/cm) and maksimum with rotogravure cylinder 48/0/130 l/cm, printing speed min 80m/min. 10,04; Drying system enables reproducible and ergonomic setting of air flow and temperatures. 10,05; Enables to read and to set drying parameters on main touchscreen panel or on surroundings of main touchscreen, to let human to operate press from one place. 11; DIE-CUTTING AND EMBOSSING SYSTEM. 11,01; Equipped with a station enabling punching and embossing during the printing process. 11,02; The station is to enable mounting of tools with different perforation pitches. 11,03; The station is to allow the assembly of tools with different shapes of embossing. 12; AUTOMATIC REGISTER SYSTEM enables:. 12,01; Continuous control and correction of the fit during machine operation over the entire speed range. 12,02; Registration accuracy – max.0,15 mm in 150 m/min during print run with all printing flexo stations and 80 m/min using three rotogravure units and the remaining flexo units. 12,03; Printing with barrier bands width max 25 mm (total). Including automatic register control system in both machine and transverse direction. 12,04; Registering when setting all units to work at a maximum of 50% of the web length in the machine. 12,05; Visibility of matt varnish application printed on 1st flexographic unit to enable the setting of registration. 12,06; Operation over the entire width of the web without additional setup operations. 13; DE-IONIZING WEB SOLUTION. 13,01; A solution for removing electrostatic loads by blowing ionized air in front of the rewinder powered by copressed air. 14; REWINDER enables:. 14,01; Rewinding of reel with diameter - at least 1000 mm. 14,02; Rewinding of reels on sleeves d76 mm and d152 mm (internal diameter). 14,03; Setting and reading rewinder parameters on main touchscreen panel or in surroundings of main touchscreen (to let human to operate press from one place). 14,04; Unloading the roll to the floor level. 14,05; Winding web pressure, with the web pressure force to be adjustable in a measurable and repeatable manner. 15; DIGITAL FEATURES enable. 15,01; Job recipe solution to save printing parameters. 15,02; Access to loop of 10 day history of machine diagnostic / maintenance alarms and errors. Data to transfer via communication port. Storage of machine operating parameters and the course of the process on the Masterpress server. 15,03; Extended maintenance information to read via main touchscreen panel (or other equivalent solution) and to contain: 1. Service alarm definition (f.e. ERROR 453) 2. Description what is suggested root cause of problem 3. What elements, part of machine need to be checked or diagnosed. 15,04; Monthly summary containing reports from the uv system including data on printing time, idle time, downtime and their % proportions. 16; VISUAL INSPECTION SYSTEM enables below features:. 16,01; Printing control within the entire print area (with x printing circumference) and different artworks on one photopolymer. 16,02; Control of: print registration, *bar code verification, *verification of master image to PDF file, * label dimensions, * printing length dimension, *stability. 16,03; Work with above values in maximum press speed. 16,04; Adjusting the illumination power of the measurement. 16,05; Work flow to transfer of information from printing machine to finishing machine including required equipment and components. 16,06; The cooperation of the press with the system: - marking sections with defects on the printed web - automatically stopping the machines in the subsequent stages of the process in order to remove the defective material. - equipped with appropriate devices and mechanisms supporting the above mentioned requirements.. 17; GENERAL REQUIREMENTS. 17,01; Installation and training of operators in Masterpress factory in the operation of the machine and auxiliary systems, daily maintenance, problem solving. 17,02; Training of the maintenance team at the Masterpress factory on potential failures, maintenance, problem solving, establishing a list of items required to secure continuous operation of the machine. 17,03; Training the IT team on the requirements and operation of the communication systems installed on the press. 17,04; DDP (Delivered Duty Paid) Białystok - the organization and costs of loading and transport to the headquarters of the commissioning party at Jacka Kuronia 4 in Bialystok, insurance and possible customs duties are borne by the Supplier (the Seller) along with the risk associated with the loss or damage of the goods.. Machine must be brand-new. Tenderer is obliged to attach to the Tender Form a detailed specification of the contract object, which includes technical parameters and functionality of the machine. The specification should contain descriptions relating to the functionality desirable in Enclosure no 1. |

|  | Printing machine with specialized convex printing unit as a part of Shrink Sleeve Label (SSL) with convex printing – 1 pcs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| press unit | U1 | U2 | U3 | U4 | U5 | U6 | U7 |  | U8 | U9 | U10 | U11 | U12 | U13 | U14 | U15 | 3D INKJET UNIT |  | VISUAL SYSTEM |  |
| Turnbar (after unit), vertical | __TB | __TB |  |  |  |  |  |  |  |  |  |  |  | __TB |  | __TB | - inkjet heads - two lane printing - additional systems (control, suppliy, communicate, data manage etc) |  | - register system, cameras on each unit - vision system: control to PDF, bar code verification, amount of control points etc. |  |
| CST installation (d 40cm) |  |  | CST | CST | CST | CST | CST |  | CST | CST | CST | CST |  |  |  |  |  |  |  |  |
| flexo | Flexo |  | Flexo | Flexo | Flexo | Flexo | Flexo |  | Flexo | Flexo | Flexo | Flexo |  | Flexo | Flexo |  |  |  |  |  |
|  | 1 |  | 2 | 3 | 4 | 5 | 6 |  | 7 | 8 | 9 | 10 |  | 11 | 12 |  |  |  |  |  |
|  | vert |  | stand | stand | stand | stand | stand |  | stand | stand | stand | stand |  | stand | vert |  |  |  |  |  |
| rotogravure |  | Roto | Roto | Roto |  | Roto | Roto |  |  | Roto | Roto |  | Roto |  |  | Roto |  |  |  |  |
|  |  | 1 | 1,2 | 1,2 |  | 2 | 2 |  |  | 2 | 2,3 |  | 2,3 |  |  | 3 |  |  |  |  |
| additional web cooling (after rotogravure unit) |  | __CHI.DR |  |  |  |  |  |  |  |  |  |  | __CHI.DR |  |  | __CHI.DR |  |  |  |  |
| drying | uv x2 |  | uv | uv | uv | uv | uv |  | uv | uv | uv | uv |  | uv x2 | uv x2 |  |  |  |  |  |
|  |  |  | led | led | led | led | led |  | led | led | led | led |  | led x2 | led x2 |  |  |  |  |  |
|  | hot air | hot air |  |  |  |  |  |  |  |  |  |  | hot air | hot air | hot air | hot air |  |  |  |  |
|  | tunnel 1 | tunnel 1 | tunnel 1 | tunnel 1,2 | - | tunnel 1,2 | tunnel 2 |  | - | tunnel 2 | tunnel 2 |  | tunnel 2,3 | tunnel 3,4 | tunnel 3,4 | tunnel 4 |  |  |  |  |
| REMARKS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| range of substrates: film 15-50um, PET, Low density, PE, PP, OPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chilling drum after last rotogravure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 roto heads (without master) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| chilling drums after roto (no extra frame needed) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Printing machine with die-cutting and embossing unit as a part of Shrink Sleeve Label (SSL) with perforation production line – 1 pcs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| press unit | U1 | U2 | U3 | U4 | U5 | U6 | U7 | U8 | U9 | U10 | U11 | DIE CUTTING EMBOSS UNIT | VISUAL SYSTEM |  |  |
| turn bar, vertical | __TB |  |  |  |  |  |  | __TB |  |  |  | to diecut peroration and make embossing | - register system, cameras on each unit - vision system: control to PDF, bar code verification, amount of control points etc. |  |  |
| cold foil (c.stamp.) |  | CST | CST | CST | CST | CST | CST | CST |  |  |  |  |  |  |  |
| flexo | Flexo | Flexo | Flexo | Flexo | Flexo | Flexo | Flexo | Flexo |  | Flexo | Flexo |  |  |  |  |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  | 9 | 10 |  |  |  |  |
|  | vert | stand | stand | stand | stand | stand | stand | stand |  | vert | vert |  |  |  |  |
| rotogravure |  |  |  |  |  |  |  | Roto | Roto |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 1 | 2 |  |  |  |  |  |  |
| additional web cooling (after roto unit) |  |  |  |  |  |  |  | __CHI.DR | __CHI.DR |  |  |  |  |  |  |
| drying | uv x2 | uv | uv | uv | uv | uv | uv | uv x2 * |  | uv x2* | uv x2 |  |  |  |  |
|  | led | led | led | led | led | led | led | led x2* |  | led x2* | led x2* |  |  |  |  |
|  | hot air |  |  |  |  |  |  | hot air | hot air | hot air | hot air |  |  |  |  |
|  | tunnel 1 |  |  |  |  |  |  | tunnel 2 | tunnel 2,3 | tunnel 3,4 | tunnel 3,4 |  |  |  |  |

|  | U1 |  |
| --- | --- | --- |
| frames | 1 | 2 |
| power supply | UV | UV |
| power supply |  | LED |

| UV and UV LED drying configuration requirements: |  |  |  | partially FL | max UV in total / UV LED max in total (power supply) |
| --- | --- | --- | --- | --- | --- |
| FL8 |  | FL9 | FL10 |  |  |
| 2 |  | 0 | 0 | 2 | 10-UV / 9-UV LED |
| 1 |  | 2 | 1 | 4 | 12-UV / 11-UV LED |
| 1 |  | 1 | 2 | 4 | 12-UV / 11-UV LED |
| 1 |  | 2 | 2 | 5 | 13-UV / 12-UV LED |
| 2 |  | 2 | 0 | 4 | 12-UV / 11-UV LED |
|  |  |  |  |  | 14 frames in total |

| REMARKS: |  |  |  |  |
| --- | --- | --- | --- | --- |
| range of substrates: film 15-50um, PET, Low density, PE, PP, OPS |  |  |  |  |
| Chilling drum on U11 |  |  |  |  |
| 2 roto heads (without master) |  |  |  |  |
| chilling drums after roto (no extra frame needed) |  |  |  |  |