- Description of composite pipe

Reinforcement layer

Inner layer (core)

Outer layer

  - Schematic diagram of a composite pipe

Fig. 1. Design record of composite pipe

  - Construction materials

Table 1. Composite pipe material list

| Inner layer | HDPE, PERT, PA |
| --- | --- |
| Reinforcement | Metal reinforced or non-metallic tape |
| Outer layer | HDPE, PERT |

HDPE – high density polyethylene,

PERT – high temperature resistant polyethylene,

PA – polyamide

- Layout

Fig.1. Schematic diagram of the technological line for applying the reinforced and external layers together with the winding and transport system.

- List of basic line devices

| No. | Item | Unit | Quantity |  |
| --- | --- | --- | --- | --- |
| 1 | Drum winder | pcs | 2 | 3 |
| 2 | Pipe centering stabilizer | pcs | 1 | 1 |
| 3 | Haul off | pcs | - | 2 |
| 4 | *Wrapping machine for reinforcing tape | set | 1 | 2 |
| 5 | *IR heater | Set | 1 | 2 |
| 6 | *Air cooling ring | set | 1 | 2 |
| 7 | Ekstruder | pcs | - | 1 |
| 8 | Cross die head | pcs | - | 1 |
| 9 | Air cooling ring after cross die | set | 1 | 1 |
| 10 | Vacuum tank | pcs | 1 | 1 |
| 11 | Cooling tank | pcs | - | 1 |
| 12 | Laser printer | pcs | - | 1 |
| 13 | Steel drum | pcs | 4 | 4 |
| 14 | Winder for packing pipes into coils | pcs | - | 1 |
| 15 | Drum transport device | pcs | 1 | 1 |

Note:

* 1 set of devices consists of 2 wrapping machines (1 pc. right turn wrapping, 1 pc. left turn wrapping), 2 IR heaters and 2 air cooling rings. One wrapping machine can wind two layers of thin glass fiber reinforced tape, therefore one wrapping cycle can give max. 4 reinforcement layers for concept I and max. 8 in concept II