### Appendix No. 1a

Detailed description and scope of the subject of the contract:

We hereby inform you that all possible trademarks, patents or origins, sources or specific processes specified in Annex 1a, which were used to characterize the products or services, are of illustrative nature only, and the ordering party allows solutions such as those described below or equivalent , provided that they meet the minimum requirements described in the request for quotation.

- Detailed description of the subject of the order

  - Device Type: Closed-loop, in-line robotic platform designed for selective conformal coating and material dispensing.

  - Configuration: Must include a loading station with a capacity of at least two magazines, a coating machine, a receiving station with a capacity of at least two magazines and a device for heating the products after the painting process.

  - A device for heating products after painting: It must have a steel housing, a capacity of at least 300 to 600 liters, shelves with a load capacity of at least 25 kg, and insulated doors with an access lock. It must ensure stable heating at a temperature of at least 60 to 100°C, humidity control, and the ability to integrate with a fume extraction system. The device must be equipped with a touchscreen, a parameter recorder, and provide ESD protection for stored products.

  - Control axes: Minimum 3 controlled axes (X, Y, Z)

  - Positioning repeatability: XY axes: ±25 microns or better.

  - Axis Resolution: At least 5 microns.

  - Maximum travel speed: In XY axes: at least 670 mm/s.

  - Working area: (with one tool): Minimum 500 mm x 525 mm (X x Y).

  - Conveyor: Integrated chain conveyor.

  - Transport width adjustment: At least 50 mm – 475 mm.

  - Communication standard: Compliant with SMEMA protocol.

  - Possibility of installation and operation: Up to 3 applicators/valves simultaneously in one cell.

  - Second independent material line: Powered by a tank with a capacity of at least 300 ml.

  - Material Handling: Wide range of material viscosities, including solvent-based coatings and high-solids materials.

  - Configurable: With spray, needle and curtain valves (film coater).

  - Programming: Offline programming possible.

  - Data registration: Process and production parameters for each painted product with user access

  - Control computer: With dedicated software for process control and path programming, integrated with the machine.

  - Possibility of integration with a fume extraction system with airflow monitoring and necessary connections

  - Marking and compliance: CE marking and compliance with SMEMA standards.

  - Vision system: For automatic recognition of reference points (fiducials).

  - Sensor: For automatic calibration of needles/tools in the X, Y, Z axes.

  - Barcode reader: Integrated barcode reader (1D/2D).

  - Lighting: Built-in UV lighting controlled by software.

  - Payment terms: preferred after installation and signing of the acceptance protocol.

  - Delivery time no more than 12 weeks.

  - Warranty not less than 12 months.

  - Documentation in Polish.

  - Transport, installation and training included in the price.

  - Launching at least two different projects at the client's premises.

I declare that the subject of the contract offered by me, offered as part of this tender, for the company Lumel Spółka Akcyjna, ul. SŁUBICKA 4, 65-127 ZIELONA GÓRA, NIP 9731024988 until 31/07/2026 , meets all the minimum and required technical parameters (obligatory) described and desired by the Ordering Party.

| Town …………….…… | on the day of ………………………….. r | ……………..………………………….. |
| --- | --- | --- |
|  |  | (signature and stamp of the offer issuer) |