Appendix No. 1

to the procedure for the purchase

and delivery of a laminator

TECHNICAL SPECIFICATIONS

Technical requirements for the machine:

- Maximum area required to assemble the equipment: 2000 x 2000 [mm].

- Possibility of laminating photopolymer on a board with maximum dimensions of 610 x 670 [mm].

- Minimum dimensions of the laminated board: 250 x 250 [mm].

- Thickness of the processed material: 0.05–6 [mm].

- Possibility  of placing photopolymer on PCB boards made of the following materials: FR4, PTFE, ceramic, etc.

- Regulated photopolymer placement speed up to 3 [m/min].

- Regulated laminating temperature up to  130 [°C].

- Regulated pressure of the laminating roll up to 5 [bar].

- Possibility of placing various types of photopolymers, including Riston, for traditional and direct UV exposition.

- Possibility of laminating Rigid and Flex PCB boards.

- The equipment should be equipped with antistatic brushes, temperature sensor on the rolls and a photopolymer cutting system.

- A presentation of the working machine or a possibility of performing adequate tests on PCB boards is required before purchase.

Note!

- The proposal should include a complete documentation of the offered machine.

- The quotation must include costs of equipment required for the normal full operation of the machine which does not constitute its integral part, e.g. computer hardware and software, etc. In the context of the procedure, the contracting authority considers the equipment to be an integral part of the machine.

- A possibility of conducting tests of the offered machine before purchase.

Warranty requirements:

- The warranty period for the subject of the proceedings should be a minimum of 24 months.

Other requirements:

- A proposal of maintenance software by the producer/supplier, minimum requirements:

  - Maintenance services by a representative in Poland for at least 12 months.

  - Regular inspections by the producer or the representative in Poland.

- The Operation and Maintenance Manual should include parts excluded from warranty.

- A set of spare parts included with the machine and spare parts availability guarantee (at least 10 years) in the post-warranty period.

- Perpetual licence for the delivered software.

- Delivery, assembly, installation and start-up of the machine.

- Training operators – specifying training level (including software use and machine operation, maintenance and calibration).

- Training maintenance workers – specifying training level and the level of maintenance workers’ involvement in the warranty and post-warranty period (including software use and machine operation, maintenance and calibration).

- Documentation in Polish –  Operation and Maintenance Manual (OMM), consisting of the following documents:

- Specification of

- the operational readiness list for the machine and equipment,

- technical data and parameters,

- operational readiness of computer hardware and software (if applicable).

- Fitting manual for:

- climate requirements,

- utilities connection,

- transport,

- location and work space requirements.

- Instructions – operation manual.

- Instructions – programming manual.

- Maintenance instructions:

- schedule and scope of inspections, maintenance + a list of maintenance products,

- list of spare and fast-wearing parts (Part List) + drawings of their placement in the machine (equipment).

- OHS manual (notes and recommendations).

- Maintenance manual:

- Diagnostic and test procedures,

- repair and maintenance procedures (including software and parameter introduction, replacing software batteries, tuning and regulation),

- list of parameters,

- list of error codes and alerts,

- block, schematic and wiring diagrams,

- hydraulic, pneumatic and kinematic diagrams.

- Specification sheets of the main subassemblies of the equipment or machine (e.g. drivers, converters, servomotors, etc.).

- EC declaration of conformity

- Hygiene certificates (if required).

- Noise emission information.

- Information on radiation emitted on the operator and exposed person, in the case when the machine can emit non-ionising radiation.

- Certificates and licences for the software used (if applicable).

- Original version of the system software (DOS, WINDOWS, etc.).

- Backup of the machine (equipment) software – application.

- Documentation of elements subject to the supervision of the Office of Technical Inspection. The Seller is obligated to deliver it to the Contracting Authority immediately after launching the procurement procedure, having confirmed its completeness with the Office of Technical Inspection.

- If there is no original manual in the Polish language, the translation should be provided with a note “Tłumaczenie instrukcji oryginalnej (Translation of the original manual)”.

- The power supply for the machines should comply with the European standard, i.e. they should be designed to be connected to the electricity grid in Poland and hold certificates required by law, in particular the EC declaration of conformity.

All new machines, tools and technical equipment should meet compliance assessment requirements confirmed with proper marking, including CE marking, declaration of compliance and required manuals, and ensure safe and healthy work conditions, taking into consideration OHS regulations and rules. All manuals and marking of the machine and control elements delivered with the machine or equipment should be in Polish.

Maintenance or repair works or procedures for the machine, tool or technical equipment cannot cause hazard to the health or life of the maintenance staff.