Appendix No. 1

for the delivery of optical lay-up station

The subject of the order is a lay-up machine for welding multi-layer PCBs using an automatic optical system for aligning individual layers with each other.

TECHNICAL SPECIFICATIONS

Technical requirements for machine:

- Ability to weld boards with maximum dimensions: 550 x 670 [mm] and thickness: 8 [mm].

- Minimum dimensions of the processed board: 356 x 406 [mm].

- Minimum thickness of the processed material: 0,05 mm.

- Maximum dimensions of the machine: 3200 x 1500 [mm].

- The device must be adapted to the technology without pin positioning of PCB layers.

- Ability to weld rigid and flex PCBs.

- The device must automatically position and place layers before welding the PCB. Automatic positioning of laminates and prepregs (e.g., based on holes) relative to each other is required.

- Positioning accuracy: +/- 15 µm or better.

- Number of welding points – not less than 4.

- Welding temperature accuracy - no worse than ±5 [°C].

- The device must have the ability to adjust the temperature and welding time.

- Vision system with at least two adjustable cameras connected to a computer and adjustable lighting.

- The device may have automatic loading and unloading of boards from the machine.

- Device compatibility with the systems used at the department is required.

- Prior to purchase, a demonstration of the machine in operation or the ability to perform adequate tests on PCB’s is required.

Remarks:

- The tender must be accompanied by a complete documentation of the proposed device.

- The valuation must include the costs of equipment that is required for the correct, complex operation of the machine, including: hardware and software, etc. These are considered by the purchaser to be an integral part of the machine.

- Possibility to test the offered device before making a purchase.

Warranty requirements:

- Guarantee: minimum 24 months.

Other requirements:

- Manufacturer/supplier service program proposal, minimum requirements:

- Service by a representative in Poland for at least 12 months.

- Periodic inspections by the manufacturer or representative in Poland.

- The DTR should specify the parts that are excluded from the warranty.

- A set of spare parts included with the device and a guarantee of availability of spare parts (min. 10 years) during the post-warranty period.

- Perpetual license for the software provided.

- Delivery, foundation, assembly and commissioning of the device.

- Operator training – please indicate of the level of training (m.in. program and machine operation, maintenance, calibration of the device).

- Maintenance service training – please indicate the level of training including interference by the maintenance service during the warranty and post-warranty period (m.in. program and machine operation, maintenance, calibration of the device).

- Documentation in Polish – technical and operational documentation (DTR), which consists of the following documents:

- Specification for:

- a list of completion of the machine,

- technical data and parameters,

- complete computer hardware and software (if applicable).

- Installation instructions in the scope of:

- climatic requirements,

- connection of utilities,

- transport,

- the requirements of the location and working space.

- Manual – operating manual.

- Manual – programming manual.

- Maintenance Instruction:

- schedule and scope of inspections, maintenance + list of means for maintenance measures,

- Spare and Wear Parts List + Layout drawings in the machine (device).

- Instruction (comments and recommendations) in the field of health and safety.

- Service Manual:

- diagnostic and test procedures,

- repair and service procedures (m.in. entering software and parameters, replacing the software backup battery, tuning and adjustment),

- a list of parameters,

- a list of fault and alarm codes,

- block, schematic and electrical diagrams,

- hydraulic, pneumatic, kinematic diagrams.

- Data sheets of the main components of the device or machine (e.g. controllers, converters, servo drives, etc.).

- CE declaration of conformity.

- Hygienic certificates (if required).

- Information on noise emissions.

- Information on radiation emitted to the operator and exposed persons when the machinery is permitted to emit non-ionizing radiation.

- Certificates and licenses for the software used (if applicable).

- Genuine version of system software (DOS, WINDOWS, etc.).

- Backup of machine (device) software – application.

- If the "original instruction" does not exist in Polish, the translation should bear the inscription "Translation of the original instruction”.

- The devices must have a power supply in accordance with the European standard, i.e. they should be adapted to the electrical network in Poland and have the attestations and certificates required by law, in particular - CE declaration of conformity.

All new machinery, tools and technical equipment should meet the requirements for conformity assessment, confirmed by appropriate marking, including CE marking, declaration of conformity and required instructions, as well as ensure safe and hygienic working conditions taking into account health and safety regulations and principles. All instructions, markings of the machine and controls supplied with the machine or equipment should be in Polish language. Service and maintenance or repair procedures for the machine, tool or technical equipment must not pose a threat to the health and life of the operating personnel..